1
51

Network Working Group                                        Vinton Cerf

Request for Comments: 675                                    Yogen Dalal

NIC: 2                                                     Carl Sunshine

INWG: 72                                                   December 1974

         SPECIFICATION OF INTERNET TRANSMISSION CONTROL PROGRAM

                         December 1974 Version

1.  INTRODUCTION

   This document describes the functions to be performed by the

   internetwork Transmission Control Program [TCP] and its interface to

   programs or users that require its services. Several basic

   assumptions are made about process to process communication and these

   are listed here without further justification. The interested reader

   is referred to [CEKA74, TOML74, BELS74, DALA74, SUNS74] for further

   discussion.

   The authors would like to acknowledge the contributions of R.

   Tomlinson (three way handshake and Initial Sequence Number

   Selection), D. Belsnes, J. Burchfiel, M. Galland, R. Kahn, D. Lloyd,

   W. Plummer, and J. Postel all of whose good ideas and counsel have

   had a beneficial effect (we hope) on this protocol design.  In the

   early phases of the design work, R. Metcalfe, A. McKenzie, H.

   Zimmerman, G. LeLann, and M. Elie were most helpful in explicating

   the various issues to be resolved. Of course, we remain responsible

   for the remaining errors and misstatements which no doubt lurk in the

   nooks and crannies of the text.

   Processes are viewed as the active elements of all HOST computers in

   a network. Even terminals and files or other I/O media are viewed as

   communicating through the use of processes. Thus, all network

   communication is viewed as inter-process communication.

   Since a process may need to distinguish among several communication

   streams between itself and another process [or processes], we imagine

   that each process may have a number of PORTs through which it

   communicates with the ports of other processes.

   Since port names are selected independently by each operating system,

   TCP, or user, they may not be unique. To provide for unique names at

   each TCP, we concatenate a NETWORK identifier, and a TCP identifier

   with a port name to create a SOCKET name which will be unique

   throughout all networks connected together.

Cerf, Dalal & Sunshine                                          [Page 1]

RFC 675              Specification of Internet TCP         December 1974

   A pair of sockets form a CONNECTION which can be used to carry data

   in either direction [i.e. full duplex]. The connection is uniquely

   identified by the <local socket, foreign socket> address pair, and

   the same local socket can participate in multiple connections to

   different foreign sockets [see Section 2.2].

   Processes exchange finite length LETTERS as a way of communicating;

   thus, letter boundaries are significant. However, the length of a

   letter may be such that it must be broken into FRAGMENTS before it

   can be transmitted to its destination. We assume that the fragments

   will normally be reassembled into a letter before being passed to the

   receiving process. Throughout this document, it is legitimate to

   assume that a fragment contains all or a part of a letter, but that a

   fragment never contains parts of more than one letter.

   We specifically assume that fragments are transmitted from Host to

   Host through means of a PACKET SWITCHING NETWORK [PSN] [ROWE70,

   POUZ73]. This assumption is probably unnecessary, since a circuit

   switched network could also be used, but for concreteness, we

   explicitly assume that the hosts are connected to one or more PACKET

   SWITCHES [PS] of a PSN [HEKA7O, POUZ74, SCWI71].

   Processes make use of the TCP by handing it letters. The TCP breaks

   these into fragments, if necessary, and then embeds each fragment in

   an INTERNETWORK PACKET. Each internetwork packet is in turn embedded

   in a LOCAL PACKET suitable for transmission from the host to one of

   its serving PS. The packet switches may perform further formatting or

   other operations to achieve the delivery of the local packet to the

   destination Host.

   The term LOCAL PACKET is used generically here to mean the formatted

   bit string exchanged between a host and a packet switch. The format

   of bit strings exchanged between the packet switches in a PSN will

   generally not be of concern to us. If an internetwork packet is

   destined for a TCP in a foreign PSN, the packet is routed to a

   GATEWAY which connects the origin PSN with an intermediate or the

   destination PSN. Routing of internetwork packets to the GATEWAY may

   be the responsibility of the source TCP or the local PSN, depending

   upon the PSN Implementation.

   One model of TCP operation is to imagine that there is a basic

   GATEWAY associated with each TCP which provides an interface to the

   local network. This basic GATEWAY performs routing and packet

   reformatting or embedding, and may also implement congestion and

   error control between the TCP and GATEWAYS at or intermediate to the

   destination TCP.

Cerf, Dalal & Sunshine                                          [Page 2]

RFC 675              Specification of Internet TCP         December 1974

   At a GATEWAY between networks, the internetwork packet is unwrapped

   from its local packet format and examined to determine through which

   network the internetwork packet should travel next. The internetwork

   packet is then wrapped in a local packet format suitable to the next

   network and passed on to a new packet switch.

   A GATEWAY is permitted to break up the fragment carried by an

   internetwork packet into smaller fragments if this is necessary for

   transmission through the next network. To do this, the GATEWAY

   produces a set of internetwork packets, each carrying a new fragment.

   The packet format is designed so that the destination TCP may treat

   fragments created by the source TCP or by intermediate GATEWAYS

   nearly identically.

   The TCP is responsible for regulating the flow of internetwork

   packets to and from the processes it serves, as a way of preventing

   its host from becoming saturated or overloaded with traffic. The TCP

   is also responsible for retransmitting unacknowledged packets, and

   for detecting duplicates. A consequence of this error

   detection/retransmission scheme is that the order of letters received

   on a given connection is also maintained [CEKA74, SUNS74]. To perform

   these functions, the TCP opens and closes connections between ports

   as described in Section 4.3. The TCP performs retransmission,

   duplicate detection, sequencing, and flow control on all

   communication among the processes it serves.

2.  The TCP INTERFACE to the USER

2.1  The TCP as a POST OFFICE

   The TCP acts in many ways like a postal service since it provides a

   way for processes to exchange letters with each other. It sometimes

   happens that a process may offer some service, but not know in

   advance what its correspondents' addresses are. The analogy can be

   drawn with a mail order house which opens a post office box which can

   accept mail from any source. Unlike the post box, however, once a

   letter from a particular correspondent arrives, a port becomes

   specific to the correspondent until the owner of the port declares

   otherwise.

   In addition to acting like a postal service, the TCP insures end-to-

   end acknowledgment, error correction, duplicate detection,

   sequencing, and flow control.

Cerf, Dalal & Sunshine                                          [Page 3]

RFC 675              Specification of Internet TCP         December 1974

2.2  Sockets and Addressing

   We have borrowed the term SOCKET from the ARPANET terminology

   [CACR70, MCKE73]. In general, a socket is the concatenation of a

   NETWORK identifier, TCP identifier, and PORT identifier. A CONNECTION

   is fully specified by the pair of SOCKETS at each end since the same

   local socket may participate in many connections to different foreign

   sockets.

   Once the connections is specified in the OPEN command [see section

   2.3.2], the TCP supplies a [short] Local Connection Name by which the

   user refers to the connection in subsequent commands. In particular

   this facilitates using connections with initially unspecified foreign

   sockets.

   TCP's are free to associate ports with processes however they choose.

   However, several basic concepts seem necessary in an implementation.

   There must be well known sockets [WKS] which the TCP associates only

   with the "appropriate" processes by some means. We envision that

   processes may "own" sockets, and that processes can only initiate

   connections on the sockets they own [means for implementing ownership

   is a local issue, but we envision a Request Port user call, or a

   method of uniquely allocating a group of ports to a given process,

   e.g. by associating the high order bits of a port name with a given

   process.]

   Once initiated, a connection may be passed to another process that

   does not own the local socket [e.g. from logger to service process].

   Strictly speaking this is a reconnection issue which might be more

   elegantly handled by a general reconnection protocol as discussed in

   section 3.3. To simplify passing a connection within a single TCP,

   such "invisible" switches may be allowed as in TENEX systems.

   Of course, each connection is associated with exactly one process,

   and any attempt to reference that connection by another process will

   be signaled as an error by the TCP. This prevents stealing data from

   or inserting data into another process' data stream.

   A connection is initiated by the rendezvous of an arriving

   internetwork packet and a waiting Transmission Control Block [TCB]

   created by a user OPEN, SEND, INTERPUPT, or RECEIVE call [see section

   2.3]. The matching of local and foreign socket identifiers determines

   when a successful connection has been initiated. The connection

   becomes established when sequence numbers have been synchronized in

   both directions as described in section 4.3.2.

Cerf, Dalal & Sunshine                                          [Page 4]

RFC 675              Specification of Internet TCP         December 1974

   It is possible to specify a socket only partially by setting the PORT

   identifier to zero or setting both the TCP and PORT identifiers to

   zero. A socket of all zero is called UNSPECIFIED. The purpose behind

   unspecified sockets is to provide a sort of "general delivery"

   facility [useful for logger type processes with well known sockets].

   There are bounds on the degree of unspecificity of socket

   identifiers. TCB's must have fully specified local sockets, although

   the foreign socket may be fully or partly unspecified. Arriving

   packets must have fully specified sockets.

   We employ the following notation:

    x.y.z = fully specified socket with x=net, y=TCP, z=port

    x.y.u = as above, but unspecified port

    x.u.u = as above, but unspecified TCP and port

    u.u.u = completely unspecified

    with respect to implementation, u = 0 [zero]

    We illustrate the principles of matching by giving all cases of

    incoming packets which match with existing TCB's. Generally, both

    the local (foreign) socket of the TCB and the foreign (local) socket

    of the packet must match.

          TCB local   TCB foreign     Packet local    Packet foreign

    (a)     a.b.c       e.f.g           e.f.g           a.b.c

    (b)     a.b.c       e.f.u           e.f.g           a.b.c

    (c)     a.b.c       e.u.u           e.f.g           a.b.c

    (d)     a.b.c       u.u.u           e.f.g           a.b.c

    There are no other legal combinations of socket identifiers which

    match. Case (d) is typical of the ARPANET well known socket idea in

    which the well known socket (a.b.c) LISTENS for a connection from

    any (u.u.u) socket. Cases (b) and (c) can be used to restrict

    matching to a particular TCP or net.

Cerf, Dalal & Sunshine                                          [Page 5]

RFC 675              Specification of Internet TCP         December 1974

2.3  TCP USER CALLS

2.3.1  A Note on Style

    The following sections functionally define the USER/TCP interface.

    The notation used is similar to most procedure or function calls in

    high level languages, but this usage is not meant to rule out trap

    type service calls [e.g. SVC's, UUO's, EMT's,...].

    The user calls described below specify the basic functions the TCP

    will perform to support interprocess communication. Individual

    implementations should define their own exact format, and may

    provide combinations or subsets of the basic functions in single

    calls. In particular, some implementations may wish to automatically

    OPEN a connection on the first SEND, RECEIVE, or INTERRUPT issued by

    the user for a given connection.

    In providing interprocess communication facilities, the TCP must not

    only accept commands, but also return information to the processes

    it serves. This communication consists of:

    (a) general information about a connection [interrupts, remote

        close, binding of unspecified foreign socket].

    (b) replies to specific user commands indicating success or various

        types of failure.

   Although the means for signaling user processes and the exact format

   of replies will vary from one implementation to another, it would

   promote common understanding and testing if a common set of codes

   were adopted. Such a set of Event Codes is described in section 2.4.

   With respect to error messages, references to "local" and "foreign"

   are ambiguous unless it is known whether these refer to the world as

   seen by the sender or receiver of the error message. The authors

   attempted several different approaches and finally settled on the

   convention that these references would be as seen by the receiver of

   the message.

2.3.2  OPEN CONNECTION

   Format: OPEN(local port, foreign socket [, timeout])

   We assume that the local TCP is aware of the identity of the

   processes it serves and will check the authority of the process to

   use the connection specified. Depending upon the implementation of

   the TCP, the source network and TCP identifiers will either be

   supplied by the TCP or by the processes that serve it [e.g. the

Cerf, Dalal & Sunshine                                          [Page 6]

RFC 675              Specification of Internet TCP         December 1974

   program which interfaces the TCP to its packet switch or the packet

   switch itself]. These considerations are the result of concern about

   security, to the extent that no TCP be able to masquerade as another

   one, and so on. Similarly, no process can masquerade as another

   without the collusion of the TCP.

   If no foreign socket is specified [i.e. the foreign socket parameter

   is 0 or not present], then this constitutes a LISTENING local socket

   which can accept communication from any foreign socket. Provision is

   also made for partial specification of foreign sockets as described

   in section 2.2.

   If the specified connection is already OPEN, an error is returned,

   otherwise a full-duplex transmission control block [TCB] is created

   and partially filled in with data from the OPEN command parameters.

   The TCB format is described in more detail in section 4.2.2.

   No network traffic is generated by the OPEN command. The first SEND

   or INTERRUPT by the local user or the foreign user will cause the TCP

   to synchronize the connection.

   The timeout, if present, permits the caller to set up a timeout for

   all letters transmitted on the connection. If a letter is not

   successfully transmitted within the timeout period, the user is

   notified and may ignore the condition [TCP will continue trying to

   transmit] or direct the TCP to close the connection. The present

   global default is 30 seconds, and connections which are set up

   without specifying another timeout will retransmit every letter for

   at least 30 seconds before notifying the user. The retransmission

   rate may vary, and is the responsibility of the TCP and not the user.

   Most likely, it will be related to the measured time for responses to

   return from letters sent.

   Depending on the TCP implementation, either a local connection name

   will be returned to the user by the TCP, or the user will specify

   this local connection name (in which case another parameter is needed

   in the call). The local connection name can then be used as a short

   hand term for the connection defined by the <local socket, foreign

   socket> pair.

   Responses from the TCP which may occur as a result of this call are

   detailed in section 2.4.

2.3.3 SEND LETTER

   Format: SEND(local connection name, buffer address, byte count, EOL

   flag [, timeout])

Cerf, Dalal & Sunshine                                          [Page 7]

RFC 675              Specification of Internet TCP         December 1974

   This call causes the data contained in the indicated user buffer to

   be sent on the indicated connection. If the connection has not been

   opened, the SEND is considered an error. Some implementations may

   allow users to SEND first, in which case an automatic OPEN would be

   done. If the calling process is not authorized to use this

   connection, an error is returned.

   If the EOL flag is set, the data is the End Of a Letter, and the EOL

   bit will be set in the last packet created from the buffer. If the

   EOL f1ag is not set, subsequent SEND's will appear as part of the

   same letter. This extended letter facility should be used sparingly

   because some TCP's may delay processing packets until an entire

   letter is received.

   If no foreign socket was specified in the OPEN, but the connection is

   established [e.g. because a listening connection has become specific

   due to a foreign letter arriving for the local port] then the

   designated letter is sent to the implied foreign socket. In general,

   users who make use of OPEN with an unspecified foreign socket can

   make use of SEND without ever explicitly knowing the foreign socket

   address.

   However, if a SEND is attempted before the foreign socket becomes

   specified, an error will be returned. Users can use the STATUS call

   to determine the status of the connection. In some implementations

   the TCP may notify the user when an unspecified socket is bound.

   If the timeout is specified, then the current default timeout for

   this connection is changed to the new one. This can affect not only

   all letters sent including and after this one, but also those which

   have not yet been sent, since the timeout is kept in the TCB and not

   associated with each letter sent. Of course, a time is maintained for

   each internetwork packet formed so as to determine how long each of

   these has been on the retransmission queue.

   In the simplest implementation, SEND would not return control to the

   sending process until either the transmission was complete or the

   timeout had been exceeded. This simple method is highly subject to

   deadlocks and is not recommended. [For example both sides of the

   connection try to do SEND's before doing any RECEIVE's.] A more

   sophisticated implementation would return immediately to allow the

   process to run concurrently with network I/O, and, furthermore, to

   allow multiple SENDs to be in progress concurrently. Multiple SENDs

   are served in first come, first served order, so the TCP will queue

   those it cannot service immediately.

Cerf, Dalal & Sunshine                                          [Page 8]

RFC 675              Specification of Internet TCP         December 1974

   NOTA BENE: In order for the process to distinguish among error or

   success indications for different letters, the buffer address should

   be returned along with the coded response to the SEND request. We

   will offer an example event code format in section 2.4, showing the

   information which should be returned to the calling process.

   The semantics of the INTERRUPT call are described later, but this

   call can have an effect on letters which have been given to the TCP

   but not yet sent. In particular, all such letters are flushed by the

   source TCP. Thus one of the responses to a SEND may be "flushed due

   to interrupt."

   Responses from the TCP which may occur as a result of this call are

   detailed in section 2.4.

2.3.4  RECEIVE LETTER

   Format: RECEIVE(local connection name, buffer address, byte count)

   This command allocates a receiving buffer associated with the

   specified connection. If no OPEN precedes this command or the calling

   process is not authorized to use this connection, an error is

   returned.

   In the simplest implementation, control would not return to the

   calling program until either a letter was received, or some error

   occurred, but this scheme is highly subject to deadlocks [see section

   2.3.3]. A more sophisticated implementation would permit several

   RECEIVE's to be outstanding at once, These would be filled as letters

   arrive. This strategy permits increased throughput, at the cost of a

   more elaborate scheme [possibly asynchronous] to notify the calling

   program that a letter has been received.

   If insufficient buffer space is given to reassemble a complete

   letter, an indication that the buffer holds a partial letter will be

   given; the buffer will be filled with as much data as it can hold.

   The remaining parts of a partly delivered letter will be placed in

   buffers as they are made available via successive RECEIVES. If a

   number of RECEIVES are outstanding, they may be filled with parts of

   a single long letter or with at most one letter each. The event codes

   associated with each RECEIVE will indicate what is contained in the

   buffer.

   To distinguish among several outstanding RECEIVES, and to take care

   of the case that a letter is smaller than the buffer supplied, the

   event code is accompanied by both a buffer pointer and a byte count

   indicating the actual length of the letter received.

Cerf, Dalal & Sunshine                                          [Page 9]

RFC 675              Specification of Internet TCP         December 1974

   The semantics of the INTERRUPT system call are discussed later, but

   this call can have an effect on outstanding RECEIVES. When the TCP

   receives an INTERRUPT, it will flush all data currently queued up

   awaiting receipt by the receiving process. If no data is waiting, but

   several buffers have been made available by anticipatory RECEIVE

   commands, these buffers are returned to the process with an error

   indicating that any data that might have been placed in those buffers

   has been flushed. This enables the receiving process to synchronize

   its RECEIVES with the interrupt. That is, the process can distinguish

   between RECEIVES issued before the receipt of the INTERRUPT and these

   issued afterwards.

   Responses from the TCP which may occur as a result of this call are

   detailed in section 2.4.

2.3.5  CLOSE CONNECTION

   Format: CLOSE(local connection name)

   This command causes the connection specified to be closed. If the

   connection is not open or the calling process is not authorized to

   use this connection, an error is returned. Any unfilled receive

   buffers or pending send buffers will be returned to the user with

   event codes indicating they were aborted due to the CLOSE. Users

   should wait for event codes for each SEND before closing the

   connection if they wish to be certain that all letters were

   successfully delivered.

   The user may CLOSE the connection at any time on his own initiative,

   or in response to various prompts from the TCP [remote close

   executed, transmission timeout exceeded, destination inaccessible].

   Because closing a connection requires communication with the foreign

   TCP, connections may remain in the closing state for a short time.

   Attempts to reopen the connection before the TCP replies to the CLOSE

   command will result in errors.

   Responses from the TCP which may occur as a result of this call are

   detailed in section 2.4.

2.3.6  INTERRUPT

   Format: INTERRUPT(local connection name)

   A special control signal is sent to the destination indicating an

   interrupt condition. This facility can be used to simulate "break"

   signals from terminals or error or completion codes from I/O devices,

   for example. The semantics of this signal to the receiving process

Cerf, Dalal & Sunshine                                         [Page 10]

RFC 675              Specification of Internet TCP         December 1974

   are unspecified. The receiving TCP will signal the interrupt to the

   receiving process immediately upon receipt, and will also flush any

   outstanding letters waiting to be delivered. Since it is possib1e to

   tell where in the letter stream this command was invoked, it is

   possible for the receiving TCP to flush only preceding data. The

   sending TCP will flush any letters pending transmission, returning a

   special error code to indicate the flush.

   If the connection is not open or the calling process is not

   authorized to use this connection, an error is returned.

   Responses from the TCP which may occur as a result of this call are

   detailed in section 2.4.

2.3.7  STATUS

   Format: STATUS(local connection name)

   This command returns a data block containing the following

   information:

    local socket, foreign socket, local connection name, receive window,

    send window, connection state, number of letters awaiting

    acknowledgment, number of letters pending receipt [including partial

    ones], default transmission timeout

    Depending on the state of the connection, some of this information

    may not be available or meaningful. If the calling process is not

    authorized to use this connection, an error is returned. This

    prevents unauthorized processes from gaining information about a

    connection.

    Responses from the TCP which may occur as a result of this call are

    detailed in section 2.4.

2.4  TCP TO USER MESSAGES

2.4.1  TYPE CODES

    All messages include a type code which identifies the type of user

    call to which the message applies. Types are:

    0 - General message, does not apply to a particular user call

    1 - Applies to OPEN

    2 - Applies to CLOSE

Cerf, Dalal & Sunshine                                         [Page 11]

RFC 675              Specification of Internet TCP         December 1974

    3 - Applies to INTERRUPT

    10 - Applies to SEND

    20 - Applies to RECEIVE

    30 - Applies to STATUS

2.4.2  MESSAGE FORMAT [notional]

    All messages include the following three fields:

      Type code

      Local connection name

      Event code

   For message types 0-3 [General, Open, Close, Interrupt] only these

   three fields are necessary.

   For message type 10 [Send] one additional field is necessary:

      Buffer address

   For message type 20 [Receive] three additional fields are necessary:

      Buffer address

      Byte count

      End-of-letter flag

   For message type 30 [status] additional data might include;

      Local socket, foreign socket

      Send window [measures buffer space at foreign TCP]

      Receive window [measures buffer space at local TCP]

      Connection state [see section 4.3.6]

      Number of letters awaiting acknowledgment

      Number of letters awaiting receipt

      Retransmission timeout

Cerf, Dalal & Sunshine                                         [Page 12]

RFC 675              Specification of Internet TCP         December 1974

2.4.3 EVENT CODES

   The event code specifies the particular event that the TCP wishes to

   communicate to the user.

   In addition to the event code, three flags may be useful to classify

   the event into major categories and facilitate event processing by

   the user:

      E flag: set if event is an error

      L/F flag: indicates whether event was generated by Local TCP, or

      Foreign TCP or network

      P/T flag: indicates whether the event is Permanent or Temporary

      [retry may succeed]

   Events are encoded into 8 bits with the high order bits set to

   indicate the state of the E, L/F, and P/T flags, respectively.

   Events specified so far are listed below with their codes and flag

   settings. A * means a flag does not apply or can take both values for

   this event. Additional events may be defined in the course of

   experimentation.

      0  0**  general success

      1  ELP  connection illegal for this process

      2  OF*  unspecified foreign socket has become bound

      3  ELP  connection not open

      4  ELT  no room for TCB

      5  ELT  foreign socket unspecified

      6  ELP  connection already open

         EFP  unacceptable SYN [or SYN/ACK] arrived at foreign

      TCP. Note: This is not a misprint, the local meaning is different

      from foreign.

      7  EFP  connection does not exist at foreign TCP

      8  EFT  foreign TCP inaccessible [may have subcases]

      9  ELT  retransmission timeout

Cerf, Dalal & Sunshine                                         [Page 13]

RFC 675              Specification of Internet TCP         December 1974

      10 E*P  buffer flushed due to interrupt

      11 OF*  interrupt to user

      12 **P  connection closing

      13 E**  general error

      14 E*P  connection reset

   Possible events for each message type are as follows:

      Type 0[general]: 2,11,12,14

      Type 1[open]: 0,1,4,6,13

      Type 2[close]: 0,1,3,13

      Type 3[interrupt]: 0,1,3,5,7,8,9,12,13

      Type 10[send]: 0,1,3,5,7,8,9,10,11,12,13

      Type 20[receive]: 0,1,3,10,12,13

      Type 30[status]: 0,1,13

   Note that events 6(foreign), 7, 8 are generated at the foreign TCP or

   in the network[s], and these same codes are used in the error field

   of the internet packet [see section 4.2.1].

3.  HIGHER LEVEL PROTOCOLS

3.1  INTRODUCTION

   It is envisioned that the TCP will be able to support higher level

   protocols efficiently. It should be easy to interface existing

   ARPANET protocols like TELNET and FTP to the TCP.

3.2  WELL KNOWN SOCKETS

   At some point, a set of well known 24 bit port numbers must be

   picked. The type of service associated with the well known ports

   might include:

      (a)  Logger

      (b)  FTP (File transfer protocol)

Cerf, Dalal & Sunshine                                         [Page 14]

RFC 675              Specification of Internet TCP         December 1974

      (c)  RJE (Remote job entry)

      (d)  Host status

      (e)  TTY Test

      (f)  HELP - descriptive, interactive system documentation

   WE RESERVE WELL KNOWN SOCKET 0 (24 bits of 0) for global messages

   destined for a particular TCP but not related to any particular

   connection. We imagine that this socket would be used for unusual TCP

   synchronization (e.g. RESET ALL) or for testing purposes (e.g.

   sending letters to TRASHCAN or ECHO). This does not conflict with the

   usage that if a socket is 0, it is unspecified, since no user can

   SEND, CLOSE, or INTERRUPT on socket 0.

3.3  RECONNECTION PROTOCOL (RCP)

   Port identifiers fall into two categories: permanent and transient.

   For example, a Logger process is generally assigned a port identifier

   that is fixed and well known. Transient processes will in general

   have ID's which are dynamically assigned.

   In the distributed processing environment of the network, two

   processes that don't have well known port identifiers may often wish

   to communicate. This can be achieved with the help of a well known

   process using a reconnection protocol. Such a protocol is briefly

   outlined using the communication facilities provided by the TCP. It

   essentially provides a mechanism by which port identifiers are

   exchanged in order to establish a connection between a pair of

   sockets.

   Such a protoco1 can be used to achieve the dynamic establishment of

   new connections in order to have multiple processes solving a problem

   cooperatively, or to provide a user process access to a server

   process via a logger, when the logger's end of the connection can not

   be invisibly passed to the server process.

   A paper on this subject by R. Schantz [SCHA74] discusses some of the

   issues associated with reconnection, and some of the ideas contained

   therein went into the design of the protocol outlined below.

   In the ARPANET, a protocol was implemented which would allow a

   process to connect to a well known socket, thus making an implicit

   request for service, and then be switched to another socket so that

   the well known socket could be freed for use by others. Since sockets

Cerf, Dalal & Sunshine                                         [Page 15]

RFC 675              Specification of Internet TCP         December 1974

   in our TCP are permitted to have connections with more than one

   foreign socket, this facility may not be explicitly needed (i.e.

   connections <A,B> and <A,C> are distinguishable).

   However. the well known socket may be in one network and the actual

   service socket(s) may be in another network (or at least in another

   TCP). Thus, the invisible switching of a connection from one port to

   another within a TCP may not be sufficient as an "Initial Connection

   Protocol". We imagine that a process wishes to use socket N1.T1.Q to

   access well known socket N2.T2.P. However, the process associated

   with socket N2.T2.P will actually start up a new process somewhere

   which will use N3.T3.S as its server socket. The N(i) and T(i) may be

   distinct or the same. The user will send to N2.T2.P the relevant user

   information such as user name, password, and account. The server will

   start up the server process and send to N1.T1.Q the actual service

   socket ldentif1er: N3.T3.S. The connection (N1.TI.Q,N2.T2.P) can then

   be closed, and the user can do a RECEIVE on (N1.T1.Q,N3.T3.S). The

   serving process can SEND on (N3.T3.S,N1.T1.Q). There are many

   variations on this scheme, some involving the user process doing a

   RECEIVE on a different socket (e.g. (N1.T1.X,U.U.U)) with the server

   doing SEND on (N3.T3.S,N1.T1.X).  Without showing all the detail of

   synchronization of sequence numbers and the like, we can illustrate

   the exchange as shown below.

      USER                             SERVER

                                       1. RECEIVE(N2.T2.P,U.U.U)

      1. SEND (N1.T1.Q,N2.T2.P)==>

                                   <== 2. SEND(N2.T2.P,N1.T1.Q)

                                          With "N3.T3.S" as data

      2. RECEIVE(N1.T1.Q,N2.T2.P)

      3. CLOSE(N1.T1.Q,N2.T2.P)==>

                                   <:= 3. CLOSE(N2.T2.P,N1.T1.Q)

      4. RECEIVE(N1.T1.Q,N3.T3.S)

                                   <== 4. SEND(N3.T3.S,N1.T1.Q)

   At this point, a connection is open between N1.T1.Q and N3.T3.S. A

   variation might be to have the user do an extra RECEIVE on

   (N1.T1.X,U.U.U) and have the data "N1.T1.X" be sent in the first user

   SEND. Then, the server can start up the real serving process and do a

Cerf, Dalal & Sunshine                                         [Page 16]

RFC 675              Specification of Internet TCP         December 1974

   SEND on (N3.T3.S,N1.T1.X) without having to send the "N3.T3.S" data

   to the user. Or perhaps both server and receiver exchange this data,

   to assure security of the ultimate connection (i.e. some wild process

   might try to connect to N1.T1.X if it is merely RECEIVING on foreign

   socket U.U.U.).

   We do not propose any specific reconnection protocol here, but leave

   this to further deliberation, since it is really a user level

   protocol issue.

4.  TCP IMPLEMENTATION

4.1  INTRODUCTION

   Conceptually, the TCP is made up of several processes. Some of these

   deal with USER/TCP commands, and others with packets arriving from

   the network. The TCP also has an internal measurement facility which

   can be activated remotely.

   Any particular TCP could be viewed in a number of ways. It could be

   implemented as an independent process, servicing many user processes.

   It could be viewed as a set of re-entrant library routines which

   share a common interface to the local PSN, and common buffer storage.

   It could even be viewed as a set of processes, some handling the

   user, some the input of packets from the net, and some the output of

   packets to the net.

4.2  TCP DATA STRUCTURES

4.2.1  INTERNETWORK PACKET FONMAT

   8 bits: Internet information

      2 bits: Reserved for local PSN use

      2 bits: Header format (11 in binary)

      4 bits: Protocol version number

   8 bits: Header length in octets (32 is the current value)

   16 bits: Length of text in octets

   32 bits: Packet sequence number

   32 bits: Acknowledgment number (i.e. sequence number of next octet

   expected).

Cerf, Dalal & Sunshine                                         [Page 17]

RFC 675              Specification of Internet TCP         December 1974

   16 bits: Window size (in octets)

   16 bits: Control Information

      Listed from high to low order:

      SYN: Request to synchronize sending sequence numbers

      ACK: There is a valid acknowledgment in the 32 bit ACK field

      FIN: Sender will stop SENDing and RECEIVEing on this connection

      DSN: The sender has stopped using sequence numbers and wants to

      initiate a new sequence number for sending.

      EOS: This packet is the end of a segment and therefore has a

      checksum in the 16 bit checksum field. If this bit is not set, the

      16 bit checksum field is to be ignored. The bit is usually set,

      but if fragmentation at a GATEWAY occurs, the packets preceding

      the last one will not have checksums, and the last packet will

      have the checksum for the entire original fragment (segment) as it

      was calculated by the sending TCP.

      EOL: This packet contains the last fragment of a letter. The EOS

      bit will always be set in this case.

      INT: The sender wants to INTERRUPT on this connection.

      XXX: six (6) unused control bits

      OD: three (3) bits of control dispatch:

         000: Null (the control octet contents should be ignored}

         001: Event Code is present in the control octet. These were

         defined in section 2.4.3.

         010: Special Functions

         011: Reject (codes as yet undefined)

         1XX: Unused

   8 bits: Control Data Octet

      If CD is 000 then this octet is to be ignored.

Cerf, Dalal & Sunshine                                         [Page 18]

RFC 675              Specification of Internet TCP         December 1974

      If CD is 001, this octet contains event codes defined in section

      2.4.3

      If CD is 010, this octet contains a special function code as

      defined below:

         0: RESET all connections between Source and Destination TCPs

         l: RESET the specific connection referenced in this packet

         2: ECHO return packet to sender with the special function code

         ECHOR (Echo Reply).

         3: QUERY Query status of connection referenced in this packet

         4: STATUS Reply to QUERY with requested status.

         5: ECHOR Echo Reply

         6: TRASH Discard packet without acknowledgment

         >6: Unused

         Note: Special function packets not pertaining to a particular

         connection [RESET all, ECHO, ECHOR, and TRASH] are normally

         sent using socket zero as described in section 3.2.

      If CD is 01l, this octet contains an as yet undefined REJECT code.

      If CD is 1XX, this octet is undefined.

   4 bits: Length of destination network address in 4 bit units (current

   value is 1)

   4 bits: Destination network address

      1010-1111 are addresses of ARPANET, UCL, CYCLADES, NPL, CADC, and

      EPSS respectively.

   16 bits: Destination TCP address

   8 bits: Padding

   4 bits: length of source network address in 4 bit units (current

   value is 1)

   4 bits: source network address (as for destination address)

Cerf, Dalal & Sunshine                                         [Page 19]

RFC 675              Specification of Internet TCP         December 1974

   16 bits: Source TCP address

   24 bits: Destination port address

   24 bits: Source port address

   16 bits: Checksum (if EOS bit is set)

4.2.2  TRANSMISSION CONTROL BLOCK

   It is highly likely that any implementation will include shared data

   structures among parts of the TCP and some asynchronous means of

   signaling users when letters have been delivered.

   One typical data structure is the Transmission Control Block (TCB)

   which is created and maintained during the lifetime of a given

   connection. The TCB contains the following information (field sizes

   are notional only and may vary from one implementation to another):

      16 bits: Local connection name

      48 bits: Local socket

      48 bits: Foreign socket

      16 bits: Receive window size in octets

      32 bits: Receive left window edge (next sequence number expected)

      16 bits: Receive packet buffer size of TCB (may be less than

      window)

      16 bits: Send window size in octets

      32 bits: Send left window edge (earliest unacknowledged octet)

      32 bits: Next packet sequence number

      16 bits: Send packet buffer size of TCB (may be less than window)

      8 bits: Connection state

         E/C - 1 if TCP has been synchronized at least once (i.e. has

         been established, else O, meaning it is closed; this bit is

         reset after FINS are exchanged and the user has done a CLOSE).

         The bit is not reset if the connection is only desynchronized

         on send or receive or both directions.

Cerf, Dalal & Sunshine                                         [Page 20]

RFC 675              Specification of Internet TCP         December 1974

         SS - SYNCed on send side (if set) else desynchronized

         SR - SYNCed on receive side (if set, else desynchronized)

   16 bits: Special flags

      S1 - SYN sent if set

      S2 - SYN verified if set

      R - SYN received if set

      Y - FIN sent if set

      C - CLOSE from local user received if set

      U - Foreign socket unspecified if set

      SDS - Send side DSN sent if set

      SDV - Send side DSN verified if set

      RDR - Receive side DSN received if set

   Initially, all bits are off [no pun intended] (i.e. SS, SR, E/C, S1,

   S2, R, F, C, SDS, SDV, RDR =0). When R is set, so is SR. When S1 and

   S2 are both set, so is SS. SR is reset when RDR is set. SS is reset

   when both SDS and SDV are set. These bits are used to keep track of

   connection state and to aid in arriving packet processing (e.g. Can

   sequence number be validated? Only if SR is set.).

   16 bits: Retransmission timeout (in eighths of a second#]

   16 bits: Head of Send buffer queue [buffers SENT from user to TCP,

   but not packetized]

   16 bits: Tail of Send buffer queue

   16 bits: Pointer to last octet packetized in partially packetized

   buffer (refers to the buffer at the head of the queue)

   16 bits: Head of Send packet queue

   16 bits: Tail of Send packet queue

   16 bits: Head of Packetized buffer Queue

   16 bits: Tail of Packetized buffer queue

Cerf, Dalal & Sunshine                                         [Page 21]

RFC 675              Specification of Internet TCP         December 1974

   16 bits: Head of Retransmit packet queue

   16 bits: Tail of Retransmit packet queue

   16 bits: Head of Receive buffer queue [queue of buffers given by user

   to RECEIVE letters, but unfilled]

   16 bits: Tail of Receive buffer queue

   16 bits: Head of Receive packet queue

   16 bits: Tail of receive packet queue

   16 bits: Pointer to last contiguous receive packet

   16 bits: Pointer to last octet filled in partly filled buffer

   16 bits: Pointer to next octet to read from partly emptied packet

      [Note: The above two pointers refer to the head of the receive

      buffer and receive packet queues respectively]

   16 bits: Forward TCB pointer

   16 bits: Backward TCB pointer

4.3  CONNECTION MANAGEMENT

4.3.1  INITIAL SEQUENCE NUMBER SELECTION

   The protocol places no restriction on a particular connection being

   used over and over again. New instances of a connection will be

   referred to as incarnations of the connection. The problem that

   arises owing to this is, "how does the TCP identify duplicate packets

   from previous incarnations of the connection?". This problem becomes

   harmfully apparent if the connection is being opened and closed in

   quick succession, or if the connection breaks with loss of memory and

   is then reestablished.

   The essence of the solution [TOML74] is that the initial sequence

   number [ISN] must be chosen so that a particular sequence number can

   never refer to an "o1d" octet, Once the connection is established the

   sequencing mechanism provided by the TCP filters out duplicates.

   For an association to be established or initialized, the two TCP's

   must synchronize on each other's initial sequence numbers. Hence the

   solution requires a suitable mechanism for picking an initial

   sequence number [ISN], and a slightly involved handshake to exchange

Cerf, Dalal & Sunshine                                         [Page 22]

RFC 675              Specification of Internet TCP         December 1974

   the ISN's. A "three way handshake" is necessary because sequence

   numbers are not tied to a global clock in the network, and TCP's may

   have different mechanisms for picking the ISN's. The receiver of the

   first SYN has no way of knowing whether the packet was an old delayed

   one or not, unless it remembers the last sequence number used on the

   connection which is not always possible, and so it must ask the

   sender to verify this SYN.

   The "three way handshake" and the advantages of a "clock-driven"

   scheme are discussed in [TOML74]. More on the subject, and algorithms

   for implementing the clock-driven scheme can be found in [DALA74].

4.3.2 ESTABLISHING A CONNECTION

   The "three way handshake" is essentially a unidirectional attempt to

   establish the connection, i.e. there is an initiator and a responder.

   The TCP's should however be able to establish the connection even if

   a simultaneous attempt is made by both TCP's to establish the

   connection. Simultaneous attempts are treated like "collisions" in

   "Aloha" systems and these conflicts are resolved into unidirectional

   attempts to establish the connection. This scheme was adopted because

      (i) Connections will normally have a passive and an active end,

      and so the mechanism should in most cases be as simple as

      possible.

      (ii) It is easy to implement as special cases do not have to be

      accounted for.

   The example below indicates what a three way handshake between TCP's

   A and B looks like

         A                                                 B

         --> <SEQ x><SYN>                                  -->

         <-- <SEQ y><SYN, ACK x+l>                         <--

         --> <SEQ x+1><ACK y+l><DATA BYTES>                -->

   The receiver of a "SYN" is able to determine whether the "SYN" was

   real (and not an old duplicate) when a positive "ACK" is returned for

   the receiver's "SYN,ACK" in response to the "SYN". The sender of a

   "SYN" gets verification on receipt of a "SYN,ACK" whose "ACK" part

   references the sequence number proposed in the original "SYN" [pun

   intended]. If the TCP is in the state where it is waiting for a

   response to its SYN, but gets a SYN instead, then it always thinks

   this is a collision and goes into the state prior to having sent the

Cerf, Dalal & Sunshine                                         [Page 23]

RFC 675              Specification of Internet TCP         December 1974

   SYN, i.e. it forgets that it had sent a SYN. The TCP will try to

   establish the connection again after some time, unless it has to

   respond to an arriving SYN. Even if the wait times in the two TCPs

   are the same, the varying delays in network transmission will usually

   be adequate to avoid a collision on the next cycle of attempts to

   send SYN.

   When establishing a connection, the state of the TCP is represented

   by 3 bits --

      S1 S2 R

      S1 = 1 -- SYN sent

      S2 = 1 -- My SYN verified

      R = 1 -- SYN received

   Some examples of attempts to establish the connection are now shown.

   The state of the connection is indicated when a change occurs. We

   specifically do not show the cases in which connection

   synchronization is carried out with packets containing both SYN and

   data. We do this to simplify the explanation, but we do not rule out

   an implementation which is capable of dealing with data arriving in

   the first packet (it has to be stored temporarily without

   acknowledgment or delivery to the user until the arriving SYN has

   been verified).

   The "three way handshake" now looks like --

              A                                            B

      ------------                                      ------------

      S1 S2 R                                                S1 S2 R

      0  0 0                                                 0  0 0

             --> <SEQ x><SYN>                           -->

      1  0 0                                                 0  0 1

             <-- <SEQ y><SYN, ACK x+l>                  <--

      1  1 1                                                 1  0 1

             --> <SEQ x+1><ACK y+1>(DATA OCTETS)        -->

      1  1 1                                                 1  1 1

Cerf, Dalal & Sunshine                                         [Page 24]

RFC 675              Specification of Internet TCP         December 1974

   The scenario for a simultaneous attempt to establish the connection

   without the arrival of any delayed duplicates is --

                    A                                     B

            ------------                               ------------

            S1 S2 R                                         S1 S2 R

             0  0 0                                          0  0 0

      (M1)   1  0 0 --> <SEQ x><SYN>                    ...

      (M2)   0  0 0 <-- <SEQ y><SYN)                    <--  1  0 0

      (M1)              B returns no SYN sent           -->  0  0 0

      (M1)   1  0 0 --> <SEQ z><SYN>      *             -->  0  0 1

      (M3)   1  1 1 <-- <SEQ y+1><SYN,ACK z+1>          <--  1  0 1

      (M4)   1  1 1 --> <SEQ z+1><ACK y+1><DATA>        -->  1  1 1

      Note: "..." means that a message does not arrive, but is delayed

      in the network. State changes are upon arrival or upon departure

      of a given message, as the case may be. Packets containing the SYN

      or INT or DSN bits implicitly contain a "dummy" data octet which

      is never delivered to the user, but which causes the packet

      sequence numbers to be incremented by 1 even if no real data is

      sent. This permits the acknowledgment of these controls without

      acknowledging receipt of any data which might also have been

      carried in the packet. A packet containing a FIN bit has a dummy

      octet following the last octet of data (if any) in the packet.

      * Once in state 000 sender selects new ISN z when attempting to

      establish the connection again.

4.3.3 HALF-OPEN CONNECTIONS

   An established connection is said to be a "half-open" connection if

   one of the TCP's has closed the connection at its end without the

   knowledge of the other, or if the two ends of the connection have

   become desynchronized owing to a crash that resulted in loss of

   memory. Such connections will automatically become reset if an

   attempt is made to send data in either direction. However, half-open

   connections are expected to be unusual, and the recovery procedure is

   somewhat involved.

Cerf, Dalal & Sunshine                                         [Page 25]

RFC 675              Specification of Internet TCP         December 1974

   If one end of the connection no longer exists, then any attempt by

   the other user to send any data on it will result in the sender

   receiving the event code "Connection does not exist at foreign TCP".

   Such an error message should indicate to the user process that

   something is wrong and it is expected to CLOSE the connection.

   Assume that two user processes A and B are communicating with one

   another when a crash occurs causing loss of memory to B's TCP.

   Depending on the operating system supporting B's TCP, it is likely

   that some error recovery mechanism exists. When the TCP is up again B

   is likely to start again from the beginning or from a recovery point.

   As a result B will probably try to OPEN the connection again or try

   to SEND on the connection it believes open. In the latter case 1t

   receives the error message "connection not open" from the local TCP.

   In an attempt to establish the connection B's TCP will send a packet

   containing SYN. A's TCP thinks that the connection is already

   established and so will respond with the error "unacceptable SYN (or

   SYN/ACK) arrived at foreign TCP". B's TCP knows that this refers to

   the SYN it just sent out, and so should reset the connection and

   inform the user process of this fact.

   It may happen that B is passive and only wants to receive data. In

   this case A's data will not reach B because the TCP at B thinks the

   connection is not established. As a result A'S TCP will timeout and

   send a QRY to B's TCP. B's TCP will send STATUS saying the connection

   is not synched. A's TCP will treat this as if an implicit CLOSE had

   occurred and tell the user process, A, that the connection is

   closing. A is expected to respond with a CLOSE command to his TCP.

   However, A's TCP does not send a FIN to B's TCP, since it would not

   be accepted anyway on the unsynced connection. Eventually A will try

   to reopen the connection or B will give up and CLOSE. If B CLOSES,

   B's TCP will simply delete the connection since it was not

   established as far as B's TCP is concerned. No message will be sent

   to A'S TCP as a result.

4.3.4  RESYNCHRONIZING A CONNECTION

   Details of resynchronization have not yet been specified since the

   need for this should be infrequent in the initial testing stages.

4.3.5 CLOSING A CONNECTION

   There are essentially three cases:

      a) The user initiates by telling the TCP to CLOSE the connection

      b) The remote TCP initiates by sending a FIN control signal

Cerf, Dalal & Sunshine                                         [Page 26]

RFC 675              Specification of Internet TCP         December 1974

      c) Both users CLOSE simultaneously

   Two bits are used to maintain control over the closing of a

   connection: these are called the "FIN sent" bit [F] and the "USER

   Closed" bit, [C] respectively. The control procedure uses these two

   bits to assure that the connection is properly closed.

   Case 1: Local user initiates the close

      In this case, both the F and C bits are initially zero, but the C

      bit is set immediately upon receipt of the user call "CLOSE." When

      the FIN is sent out by the TCP, the F bit is set. All pending

      RECEIVES are terminated and the user is told that they have been

      prematurely terminated ("connection closing"} without data.

      Similarly, any pending SENDS are terminated with the same

      response, "connection closing."

      Several responses may arrive as the result of sending a FIN. The

      one which is generally expected is a matching FIN. When this is

      received, the TCB CAN BE ELIMINATED. If a "connection does not

      exist at foreign TCP" message comes in response to the FIN, then

      the TCB can likewise be eliminated. If no response is forthcoming,

      or if "Foreign TCP inaccessible" arrives then the resolution is

      moot. One might simply timeout and discard the TCB. Since the

      local user wants to CLOSE anyway, this is probably satisfactory,

      although it will leave a potential "half-open" connection at the

      other side. We deal with half open connections in section 4.3.3.

      When the acknowledging FIN arrives after the connection state bits

      are set (F=1, C=1), then the TCB can be deleted.

   Case 2: TCP receives a FIN from the network

      First of all, a FIN must have a sequence number which lies in the

      valid receive window. If not, it is discarded and the left window

      edge is sent as acknowledgment. If the FIN can be processed, it is

      handled (possibly out of order, since it is taken as an imperative

      to shut down the connection). All pending RECEIVES and SENDS are

      responded to by showing that they were terminated by the other

      side's close request (i.e. "connection closing"). The user is also

      told by an unsolicited event or signal that the connection has

      been closed (in some systems, the user might have to request

      STATUS to get this information). Finally, the TCP sends FIN in

      response.

      Thus, because a FIN arrived, a FIN is sent back, so the F bit is

      set. However, the TCB stays around until the local user does a

      CLOSE in acknowledgment of the unsolicited signal that the

Cerf, Dalal & Sunshine                                         [Page 27]

RFC 675              Specification of Internet TCP         December 1974

      connection has been closed by the other side. Thus, the C bit

      remains unset until this happens. If the C and F bits go from (F=1

      C=O) to (F=l, C=1), then the connection is closed and the TCB can

      be removed.

   Case 3: both users close simultaneously

      If this happens, both connections will be in the (F=1, C=1) state.

      When the FINs arrive, the connections w11i be shut down. If one

      FIN fails to arrive, we have two choices. One is to insist on

      acknowledgments for FINs, in which case the missing one will be

      retransmitted. Another is merely to permit the half-open

      connection to remain (we prefer this solution}. It can timeout

      independently and go away after a while. If an attempt is made to

      reestablish the connection, the initiator will discover the

      existence of the open connection since an "inappropriate SYN

      received" message will be sent by the TCP which holds the "half-

      open" connection. The receiver of this message can tell the other

      TCP to reset the connection. We cannot permit the holder of the

      half-open connection to reset automatically on receipt of the SYN

      since its receipt is not necessarily prima facie evidence of a

      half open connection. (The SYN could be a delayed duplicate.)

4.3.6.  CONNECTION STATE and its relation to USER and INCOMING CONTROL

   REQUESTS

   In order to formalize the action taken by the TCP when it receives

   commands from the User, or Control information from the network, we

   define a connection to be in one of 7 states at any instant. These

   are known as the TCB Major States. Each Major State is simply a

   convenient name for a particular setting or group of settings of the

   state bits, as follows:

      S1 S2  R  U  F  C   #   name

       -  -  -  -  -  -   0   no TCB

       0  0  0 0/1 0  0   1   unsync

       1  0  0  0  0  0   2   SYN sent

       1  0  1 0/1 0  0   3   SYN received

       1  1  1  0  0  0   4   established

       1 0/1 1 0/1 1  1   5   FIN wait

       1  1  1  0  1  0   6   FIN received

Cerf, Dalal & Sunshine                                         [Page 28]

RFC 675              Specification of Internet TCP         December 1974

   The connection moves from state to state as shown below. The

   transition from one state to another will be represented as

      [X, Y]<cause><action>

   which means that there is a transition from state X to state Y owing

   to <cause>. The action taken by the TCP is specified as <action>. We

   use this notation to give the important state transitions, often

   simplifying the cause and action fields to take into account a number

   of situations. Figure 1 illustrates these transitions in traditional

   state diagram form. Section 4.4.6 and section 4.4.7 fully specify the

   effect of all User commands and Control information arriving from the

   network.

      [0,l] <OPEN> <create TCB>

      [1,2] <SEND,INTERRUPT, or collision timeout> <send SYN>

      [1,3] <SYN arrives> <send SYN,ACK>

      [1,0] <CLOSE> <remove TCB>

      [2,1] <SYN arrives (collision)> <set timeout, forget SYNs>

      [2,0] <CLOSE> <remove TCB>

      [2,4] <appropriate SYN,ACK arrives> <send ACK>

      [3,4] <appropriate ACK arrives> <none>

      [3,1] <error arrives or timeout> <(forget SYN)>

      [3,5] <CLOSE> <send FIN>

      [4,5] <CLOSE> <send FIN>

      [4,6] <appropriate FIN arrives> <send FIN, inform user>

      [5,0] <FIN or error arrives, or timeout> <remove TCB>

      [6,0] <CLOSE> <remove TCB>

4.4  STRUCTURE 0F THE TCP

4.4.l  INTRODUCTION [See figure 2.1]

   There are many possible implementations of the TCP. We offer one

   conceptual framework in which to view the various algorithms that

Cerf, Dalal & Sunshine                                         [Page 29]

RFC 675              Specification of Internet TCP         December 1974

   make up the TCP design. In our concept, the TCP is written in two

   parts, an interrupt or signal driven part (consisting of four

   processes), and a reentrant library of subroutines or system calls

   which interface the user process to the TCP. The subroutines

   communicate with the interrupt part through shared data structures

   (TCB's, shared buffer queues etc.). The four processes are the Output

   Packet Handler which sends packets to the packet switch; the

   Packetizer which formats letters into internet packets; the Input

   Packet Handler which processes incoming packets; and the Reassembler

   which builds letters for users.

   The ultimate bottleneck is the pipe through which arriving and

   departing packets must travel. This is the Host/Packet Switch

   interface. The interrupt driven TCP shares among all TCB's its

   limited packet buffer resources for sending and receiving packets.

   From the standpoint of controlling buffer congestion, it appears

   better to TREAT INCOMING PACKETS WITH HIGHER PRIORITY THAN OUTGOING

   PACKETS. That is, packet buffers which can be released by copying

   their contents into user buffers clearly help to reduce congestion.

   Neither the packetizer nor the input packet handler should be allowed

   to take up all available packet buffer space; an analogous problem

   arises in the IMP in the allocation of store and forward, and

   reassembly buffer space. One policy is to permit neither contender

   more than, say, two-thirds of the space. The buffer allocation

   routines can enforce these limits and reject buffer requests as

   needed. Conceptually, the scheduler can monitor the amounts of

   storage dedicated to the input and output routines, and can force

   either to sleep if its buffer allocation exceeds the limit.

   As an example, we can consider what happens when a user executes a

   SEND call to the TCP service routines. The buffer containing the

   letter is placed on a SEND buffer queue associated with the user's

   TCB. A 'packetizer' process is awakened to look through all the TCB's

   for 'packetizing' work. The packetizer will keep a roving pointer

   through the TCB list which enables it to pick up new buffers from the

   TCB queue and packetize them into output buffers. The packetizer

   takes no more than one letter at a time from any single TCB. The

   packetizer attempts to maintain a non-empty queue of output packets

   so that the output handler will not fall idle waiting for the

   packetizing operation. However, since arriving packets compete with

   departing packets, care must be taken to prevent either class from

   occupying all of the shared packet buffer space. Similarly since the

   TCB's all compete for space in service to their connections, neither

   input nor output packet space should be dominated by any one TCB.

   When a packet is created, it is placed on a FIFO SEND packet queue

   associated with its origin TCB. The packetizer wakes the output

   handler and then continues to packetize a few more buffers, perhaps,

Cerf, Dalal & Sunshine                                         [Page 30]

RFC 675              Specification of Internet TCP         December 1974

   before going to sleep. The output handler is awakened either by a

   'hungry' packet switch or by the packetizer; in either case, it uses

   a roving TCB pointer to select the next TCB for service. The send

   packet queue can be used as a 'work queue' for the output handler.

   After a packet has been sent, but usually before an ACK is returned,

   the output handler moves the packet to a retransmission queue

   associated with each TCB.

   Retransmission timeouts can refer to specific packets and the

   retransmission list can be searched for the specific packet. If an

   ACK is received, the retransmission entry can be removed from the

   retransmit queue. The send packet queue contains only packets waiting

   to be sent for the first time. INTERRUPT requests can remove entries

   in both the send packet queue and the retransmit packet queue.

   Since packets are never in more than one queue at a time, it appears

   possible for INT, FIN or RESET commands to remove packets from the

   receive, send, or retransmit packet queues with the assurance that an

   already issued signal to enter the reassembler, the packetizer or the

   output handler will not be confusing.

   Handling the INTERRUPT and CLOSE functions can however require some

   care to avoid confusing the scheduler, and the various processes. The

   scheduler must maintain status information for the processes. This

   information includes the current TCB being serviced. When an

   INTERRUPT is issued by a local process, the output queue of letters

   associated with the local port reference is to be deleted. The

   packetizer, for example, may however be working at that time on the

   same queue. As usual, simultaneous reading and writing of the TCB

   queue pointers must be inhibited through some sort of semaphore or

   lockout mechanism. When the packetizer wants to serve the next send

   buffer queue, it must lock out all other access to the queue, remove

   the head of the queue (assuming of course that there are enough

   buffers for packetization), advance the head of the queue, and then

   unlock access to the queue.

   If the packetizer keeps only a TCB pointer in a global place called

   CPTCB (current packetizer TCB address), and always uses the address

   in CPTCB to find the TCB in which to examine the send buffer queue,

   then removal of the output buffer queue does not require changes to

   any working storage belonging to the packetizer. Even more important,

   the arrival and processing of a RESET or CLOSE, which clears the

   system of a given TCB, can update the CPTCB pointer, as long as the

   removal does not occur while the packetizer is still working on the

   TCB.

Cerf, Dalal & Sunshine                                         [Page 31]

RFC 675              Specification of Internet TCP         December 1974

   Incoming packets are examined by the input packet handler. Here they

   are checked for valid connection sockets, and acknowledgments are

   processed, causing packets to be removed, possibly, from the SEND or

   RETRANSMIT packet queues as needed. As an example, consider the

   receipt of a valid FIN request on a particular TCB. If a FIN had not

   been sent before (i.e. F bit not set), then a FIN packet is

   constructed and sent after having cleared out the SEND buffer and

   SEND packet queues as well as the RETRANSMIT queue. Otherwise, if the

   F and C bits are both set, all queues are emptied and the TCB is

   returned to free storage.

   Packets which should be reassembled into letters and sent to users

   are queued by the input packet handler, on the receive packet queue,

   for processing by the reassembly process. The reassembler looks at

   its FIFO work queue and tries to move packets into user buffers which

   are queued up in an input buffer queue on each TCB. If a packet has

   arrived out of order, it can be queued for processing in the correct

   sequence. Each time a packet is moved into a user buffer, the left

   window edge of the receiving TCB is moved to the right so that

   outgoing packets can carry the correct ACK information. If the SEND

   buffer queue is empty, then the reassembler creates a packet to carry

   the ACK.

   As packets are moved 1nto buffers and they are filled, the buffers

   are dequeued from the RECEIVE buffer queue and passed to the user.

   The reassembler can also be awakened by the RECEIVE user call should

   it have a non-empty receive packet queue with an empty RECEIVE buffer

   queue. The awakened reassembler goes to work on each TCB, keeping a

   roving pointer, and sleeping if a cycle is made of all TCB's without

   finding any work.

4.4.2  INPUT PACKET HANDLER [See figure 2.2]

   The Input Packet Handler is awakened when a packet arrives from the

   network. It first verifies that the packet is for an existing TCB

   (i.e. the local and foreign socket numbers are matched with those of

   existing TCB's). If this fails, an error message is constructed and

   queued on the send packet queue of a dummy TCB. A signal is also sent

   to the output packet handler. Generally, things to be transmitted

   from the dummy TCB have a default retransmission timeout of zero, and

   will not be retransmitted. (We use the idea of a dummy TCB so that

   all packets containing errors, or RESET can be sent by the output

   packet handler, instead of having the originator of them interface to

   the net. These packets, it will be noticed, do not belong to any

   TCB).

Cerf, Dalal & Sunshine                                         [Page 32]

RFC 675              Specification of Internet TCP         December 1974

   The input packet handler looks out for control or error information

   and acts appropriately. Section 4.4.7 discusses this in greater

   detail, but as an example, if the incoming packet is a RESET request

   of any kind (i.e. all connections from designated TCP or given

   connection), and is believable, then the input packet handler clears

   out the related TCB(s), empties the send and receive packet queues,

   and prepares error returns for outstanding user SEND(s) and

   RECEIVE(s) on each reset TCB. The TCB's are marked unused and

   returned to storage. If the RESET refers to an unknown connection, it

   is ignored.

   Any ACK's contained in incoming packets are used to update the send

   left window edge, and to remove the ACK'ed packets from the TCB

   retransmit packet queue. If the packet being removed was the end of a

   user buffer, then the buffer must be dequeued from the packetized

   buffer queue, and the User informed. The packetizer is also signaled.

   Only one signal, or one for each packet, will have to be sent,

   depending on the scheduling scheme for the processes. See section

   4.4.7 for a detailed discussion.

   The packet sequence number, the current receive window size, and the

   receive left window edge determine whether the packet lies within the

   window or outside of it.

      Let W = window size

         S = size of sequence number space

         L = left window edge

         R = L+W-1 = right window edge

         x = sequence number to be tested

      For any sequence number, x, if

         (R-x) mod S <= W

      then x is within the window.

   A packet should be rejected only if all of it lies outside the

   window. This is easily tested by letting x be, first the packet

   sequence number, and then the sum of packet sequence number and

   packet text length, less one. If the packet lies outside the window,

   and there are no packets waiting to be sent, then the input packet

   handler should construct a dummy ACK and queue it for output on the

Cerf, Dalal & Sunshine                                         [Page 33]

RFC 675              Specification of Internet TCP         December 1974

   send packet queue, and signal the output packet handler. Successfully

   received packets are placed on the receive packet queue in the

   appropriate sequence order, and the reassembler signaled.

   The packet window check can not be made if the associated TCB is not

   in the 'established' state, so care must be taken to check for

   control and TCB state before doing the window check.

4.4.3  REASSEMBLER [See figure 2.3]

   The Reassembler process is activated by both the Input Packet Handler

   and the RECEIVE user call. While the reassembler is asleep, if

   multiple signals arrive, all but one can be discarded. This is

   important as the reassembler does not know the source of the signal.

   This is so in order that "dangling" signals from work in TCB's that

   have subsequently been removed don't confuse it. Each signal simply

   means that there may be work to be done. If the reassembler is awake

   when a signal arrives, it may be necessary to put 1t in a

   "hyperawake" state so that even if the reassembler tries to quit, the

   scheduler will run it one more time.

   When the reassembler is awakened it looks at the receive packet queue

   for each TCB. If there are some packets there then it sees whether

   the RECEIVE buffer queue is empty. If it is then the reassembler

   gives up on this TCB and goes on to the next one, otherwise if the

   first packet matches the left window edge, then the packet can be

   moved into the User's buffer. The reassembler keeps transferring

   packets into the User's buffer until the letter is completely

   transferred, or something causes it to stop. Note that a buffer may

   be partly filled and then a sequence 'hole' is encountered in the

   receive packet queue. The reassembler must mark progress so that the

   buffer can be filled up starting at the right place when the 'hole'

   is filled. Similarly a packet might be only partially emptied when a

   buffer is filled, so progress in the packet must be marked.

   If a letter was successfully transferred to a User buffer then the

   reassembler signals the User that a letter has arrived and dequeues

   the buffer associated with it from the TCB RECEIVE buffer queue. If

   the buffer is filled then the User is signaled and the buffer

   dequeued as before. The event code indicates whether the buffer

   contains all or part of a letter, as described in section 2.4.

   In every case when a packet is delivered to a buffer, the receive

   left window edge is updated, and the packetizer is signaled. This

   updating must take account of the extra octet included in the

   sequencing for certain control functions [SYN, INT, FIN, DSN]. If the

   send packet queue is empty then the reassembler must create a packet

   to carry the ACK, and place it on the send packet queue.

Cerf, Dalal & Sunshine                                         [Page 34]

RFC 675              Specification of Internet TCP         December 1974

   Note that the reassembler never works on a TCB for more than one User

   buffer's worth of time, in order to give all TCB's equal service.

   Scheduling of the reassembler is a big issue, but perhaps running to

   completion will be satisfactory, or else it can be time sliced. In

   the latter case it will continue from where it left off, but a new

   signal may have arrived producing some possible work. This work will

   be processed as part of the old incomplete signal, and so some

   wasteful processing may occur when the reassembler wakes up again.

   This is the general problem of trying to implement a protocol that is

   fundamentally asynchronous, but at least it is immune to harmful

   race-conditions. E.g. if we were to have the reassembler 'remove' the

   signal that caused it to wake up, just before it went to sleep (in

   order that new arriving ones were discarded) then a new signal may

   arrive at a critical time causing 1t not to be recognized; thus

   leaving some work pending, and this may result in a deadlock [see

   previous comments on "hyperawake" state].

4.4.4  PACKETIZER [See figure 2.4]

   The Packetizer process gets work from both the Input Packet Handler

   and the SEND user call. The signal from the SEND user call indicates

   that there is something new to send, while the one from the input

   packet handler indicates that more TCP buffers may be available from

   delivered packets. This latter signal is to prevent deadlocks in

   certain kind of scheduling schemes. We assume the same treatment of

   signals as discussed in section 4.4.3.

   When the packetizer is awakened it looks at the SEND buffer queue for

   each TCB. If there is a new or partial letter awaiting packetization,

   it tries to packetize the letter, TCB buffer and window permitting.

   It packetizes no more than one letter for a TCB before servicing

   another TCB. For every packet produced it signals the output packet

   handler (to prevent deadlock in a time sliced scheduling scheme). If

   a 'run till completion' scheme is used then one signal only need be

   produced, the first time a packet is produced since awakening. If

   packetization is not possible the packetizer goes on to the next TCB.

   If a partial buffer was transferred then the packetizer must mark

   progress in the SEND buffer queue. Completely packetized buffers are

   dequeued from the SEND buffer queue, and placed on a Packetized

   buffer queue, so that the buffer can be returned to the user when an

   ACK for the last bit is received.

   When the packetizer packetizes a letter it must see whether it is the

   first piece of data being sent on the connection, in which case it

   must include the SYN bit. Some implementations may not permit data to

   be sent with SYN and others may discard any data received with SYN.

Cerf, Dalal & Sunshine                                         [Page 35]

RFC 675              Specification of Internet TCP         December 1974

   The Packetizer goes to sleep if it finds no more work at any TCB.

4.4.5  OUTPUT PACKET HANDLER [see figure 2.5]

   When activated by the packetizer, or the input packet handler, or

   some of the user call routines, the Output Packet Handler attempts to

   transmit packets on the net (may involve going through some other

   network interface program). It looks at the TCB's in turn,

   transmitting some packets from the send packet queue. These are

   dequeued and put on the retransmit queue along with the time when

   they should be retransmitted.

   All data packets that are transmitted have the latest receive left

   window edge in the ACK field. Error and control messages may have no

   ACK [ACK bit off], or set the ACK field to refer to a received

   packet's sequence number.

   The RETRANSMIT PROCESS:

   This process can either be viewed as a separate process, or as part

   of the output packet handler. Its implementation can vary; it could

   either perform its function, by being woken up at regular intervals,

   or when the retransmission time occurs for every packet put on the

   retransmit queue. In the first case the retransmit queue for each TCB

   is examined to see if there is anything to retransmit. If there is, a

   packet is placed on the send packet queue of the corresponding TCB.

   The output packet handler is also signaled.

   Another "demon" process monitors all user Send buffers and

   retransmittable control messages sent on each connection, but not yet

   acknowledged. If the global retransmission timeout is exceeded for

   any of these, the User is notified and he may choose to continue or

   close the connection. A QUERY packet may also be sent to ascertain

   the state of the connection [this facilitates recovery from half open

   connections as described in section 4.3.3].

4.4.6  USER CALL PROCESSING

   OPEN [See figure 3.1]

      1. If the process calling does not own the specified local socket,

      return with <type 1><ELP 1 "connection illegal for this process">.

      2. If no foreign socket is specified, construct a new TCB and add

      it to the list of existing TCB's. Select a new local connection

      name and return it along with <type 1><OLP 0 "success">. If there

      is no room for the TCB, respond with <type 1><ELT 4 "No room for

      TCB">.

Cerf, Dalal & Sunshine                                         [Page 36]

RFC 675              Specification of Internet TCP         December 1974

      3. If a foreign socket is specified, verify that there is no

      existing TCB with the same <local socket, foreign socket> pair

      (i.e. same connection), otherwise return <type l><ELP 6

      "connection already open">. If there is no TCB space, return as in

      (2), otherwise, create the TCB and link it with the others,

      returning a local connection name with the success event code.

      Note: if a TCB is created, be sure to copy the timeout parameter

      into it, and set the "U" bit to 0 if a foreign socket is

      specified, else set U to 1 (to show unspecified foreign socket).

   SEND [see figure 3.2]

      1. Search for TCB with local connection name specified. If none

      found, return <type 10><ELP 3 "connection not open">

      2. If TCB is found, check foreign socket specification. If not set

      (i.e. U = 1 in TCB), return <type 10><ELT 5 "foreign socket

      unspecified">. If the connection is in the "closing" state (i.e.

      state 5 or 6), return <type 3><ELP 12 "connection closing"> and do

      not process the buffer.

      3. Put the buffer on the Send buffer queue and signal the

      packetizer that there is work to do.

   INTERRUPT [see figure 3.3]

      1. Validate existence of the referenced connection, sending out

      error messages of the form <type 3><ELP 3 "connection not open">

      or <type 3><ELT 5 "foreign socket unspecified"> as appropriate. If

      the local connection refers to a connection not accessible to the

      process interrupting, send <type 3><ELP 1 "connection illegal for

      this process">.

      2. If the connection is in the "closing" state (i.e. states 5 or

      6), return <type 3><ELT 12 "connection closing"> and do not send

      an INT packet to the destination.

      3. Any pending SEND buffers should be returned with <type 10><ELP

      10 "buffer flushed due to interrupt">. An INT packet should be

      created and placed on the output packet queue, and the output

      packet handler should be signaled.

   RECEIVE [See figure 3.4]

      1. If the caller does not have access to the referenced local

      connection name, return <type 20><ELP 1 "connection illegal for

      this process">. And if the connection is not open, return <type

Cerf, Dalal & Sunshine                                         [Page 37]

RFC 675              Specification of Internet TCP         December 1974

      20><ELP 3 "connection not open"). If the connection is in the

      closing state (e.g. a FIN has been received or a user CLOSE is

      being processed), return <type 20><ELP 12 "connection closing">.

      2. Otherwise, put the buffer on the receive buffer queue and

      signal the reassembler that buffer space is available.

   CLOSE [See figure 3.5]

      1. If the connection is not accessible to the caller, return <type

      2><ELP 1 "connection illegal for this process">. If there is no

      such connection respond with <type 2><ELP 3 "connection not

      open">.

      2. If the R bit is 0 (i.e. connection is in state 1 or 2), simply

      remove the TCB.

      3. If the R bit is set and the F bit is set, then remove the TCB.

      4. Otherwise, if the R bit is set, but F is 0 (i.e. states 3 or

      4), return all buffers to the User with <type x><ELP 12

      "connection closing">, clear all output and input packet queues

      for this connection, create a FIN packet, and signal the output

      packet handler. Set the C and F bits to show this action.

   STATUS [See figure 3.6]

      1. If the connection is illegal for the caller to access, send

      <type 30><ELP 1 "connection illegal for this process">.

      2. If the connection does not exist, return <type 30><ELP 3

      "connection not open">.

      3. Otherwise set status information from the TCB and return it via

      <type 30><O-T 0 "status data...">.

4.4.7  NETWORK CONTROL PROCESSING

   The Input Packet Handler examines the header to see if there is any

   control information or error codes present. We do not discuss the

   action taken for various special function codes, as it is often

   implementation dependent, but we describe those that affect the state

   of the connection. After initial screening by the IPC [see section

   4.4.2 and figure 2.2], control and error packets are processed as

   shown in figures 4.l-4.7. [ACK and data processing is done within the

   IPC.]

Cerf, Dalal & Sunshine                                         [Page 38]

RFC 675              Specification of Internet TCP         December 1974

4.4.8  TCP ERROR HANDLING

   Error messages have CD=001 and do not carry user data. Depending on

   the error, zero or more octets of error information will be carried

   in the packet text field. We explicitly assume that this data is

   restricted in length so as to fall below the GATEWAY fragmentation

   threshold (probably 512 bits of data and header). Errors generally

   refer to specific connections, so the source and destination socket

   identifiers are relevant here. The ACK field of an error packet

   contains the sequence number of the packet that caused the error, and

   the ACK bit is off. [RESET and STATUS special functions may use the

   ACK field in the same way.] This allows the receiver of an error

   message to determine which packet caused the error. Error packets are

   not ACK'ed or retransmitted.

4.5.  BUFFER AND WINDOW ALLOCATION

4.5.1  INTRODUCTION

   The TCP manages buffer and window allocation on connections for two

   main purposes: equitably sharing limited TCP buffer space among all

   connections (multiplexing function), and limiting attempts to send

   packets, so that the receiver is not swamped (flow control function).

   For further details on the operation and advantages of the window

   mechanism see CEKA74.

   Good allocation schemes are one of the hardest problems of TCP

   design, and much experimentation must be done to develop efficient

   and effective algorithms. Hence the following suggestions are merely

   initial thoughts. Different implementations are encouraged with the

   hope that results can be compared and better schemes developed.

   Several of the measurements discussed in a later section are aimed at

   providing information on the performance of allocation mechanisms.

   This should aid in determining significant parameters and evaluating

   alternate schemes.

4.5.2 The SEND Side

   The window is determined by the receiver. Currently the sender has no

   control over the SEND window size, and never transmits beyond the

   right window edge. There exists the possibility of specifying two

   more special function codes so that the sender can request the

   receiver to INCREASE or DECREASE the window size, without specifying

   by how much. The receiver, of course, needn't satisfy this request.

Cerf, Dalal & Sunshine                                         [Page 39]

RFC 675              Specification of Internet TCP         December 1974

   Buffers must be allocated for outgoing packets from a TCP buffer

   pool. The TCP may not be willing to allocate a full window's worth of

   buffers, so buffer space for a connection may be less than what the

   window would permit. No deadlocks are possible even if there is

   insufficient buffer or window space for one letter, since the

   receiver will ACK parts of letters as they are put into the user's

   buffer, thus advancing the window and freeing buffers for the

   remainder of the letter.

   It is not mandatory that the TCP buffer outgoing packets until

   acknowledgments for them are received, since it is possible to

   reconstruct them from the actual letters sent by the user.

   However, for purposes of retransmission and processing efficiency it

   is very convenient to do.

4.5.3  The RECEIVE Side

   At the receiving side there are two requirements for buffering:

   (l) Rate Discrepancy:

      If the sender produces data much faster or much slower than the

      receiver consumes it, little buffering is needed to maintain the

      receiver at near maximum rate of operation. Simple queuing

      analysis indicates that when the production and consumption

      (arrival and service) rates are similar in magnitude, more

      buffering is needed to reduce the effect of stochastic or bursty

      arrivals and to keep the receiver busy.

   (2) Disorderly Arrivals:

      When packets arrive out of order, they must be buffered until the

      missing packets arrive so that packets (or letters) are delivered

      in sequence. We do not advocate the philosophy that they be

      discarded, unless they have to be, otherwise a poor effective

      bandwidth may be observed. Path length, packet size, traffic

      level, routing, timeouts, window size, and other factors affect

      the amount by which packets come out of order. This is expected to

      be a major area of investigation.

   The considerations for choosing an appropriate window are as follows:

   Suppose that the receiver knows the sender's retransmission timeout,

   also, that the receiver's acceptance rate is 'U' bits/sec, and the

   window size is 'W' bits. Ignoring line errors and other traffic, the

   sender transmits at a rate between W/K and the maximum line rate (the

   sender can send a window's worth of data each timeout period).

Cerf, Dalal & Sunshine                                         [Page 40]

RFC 675              Specification of Internet TCP         December 1974

   If W/K is greater than U, the difference must be retransmissions

   which is undesirable, so the window should be reduced to W', such

   that W'/K is approximately equal to U. This may mean that the entire

   bandwidth of the transmission channel is not being used, but it is

   the fastest rate at which the receiver is accepting data, and the

   line capacity is free for other users. This is exactly the same case

   where the rates of the sender and receiver were almost equal, and so

   more buffering is needed. Thus we see that line utilization and

   retransmissions can be traded off against buffering.

   If the receiver does not accept data fast enough (by not performing

   sufficient RECEIVES) the sender may continue retransmitting since

   unaccepted data will not be ACK'ed. In this case the receiver should

   reduce the window size to "throttle" the sender and inhibit useless

   retransmissions.

   Receiver window control:

      If the user at the receiving side is not accepting data, the

      window should be reduced to zero. In particular, if all TCP

      incoming packet buffers for a connection are filled with received

      packets, the window must go to zero to prevent retransmissions

      until the user accepts some packets.

      Short term flow control:

      Let F = the number of user receive buffers filled

         B = the total user receive buffers

         W = the long-term or nominal window size

         W' = the window size returned to the sender

      then a possible value for W' is

         W' = W*[1-F/B]**a

      The value of 'a' should be greater than one, in order to shut the

      window faster as buffers run out. The values of W' and F actually

      used could be averages of recent values, in order to get smooth

      control. Note that W' is constantly being recomputed, while the

      value of W, which sets the upper limit of W', only changes slowly

      in response to other factors.

      The value of W can be large (up to half the sequence number space)

      to allow for good throughput on high delay channels. The sender

      needn't allocate W worth of buffer space anyway. The long-term

Cerf, Dalal & Sunshine                                         [Page 41]

RFC 675              Specification of Internet TCP         December 1974

      variation of W to match flow requirements may be a separate

      question

   This short-term mechanism for flow control allows some buffering in

   the two TCP's at either end, (as much as they are willing), and the

   rest in the user process at the send side where the data is being

   created. Hence the cost of buffering to smooth out bursty traffic is

   borne partly by the TCP's, and partly by the user at the send side.

   None of it is borne by the communication subnet.

5.  NETWORK MEASUREMENT PLANS FOR TCP

5.1  USERLEVEL DIAGNOSTICS

   We have in mind a program which will exercise a given TCP, causing it

   to cycle through a number of states; opening, closing, and

   transmitting on a variety of connections. This program will collect

   statistics and will generally try to detect deviation from TCP

   functional specifications. Clearly there will have to be a copy of

   this program both at the local site being tested and some site which

   has a certified TCP. So we will have to produce a specification for

   this user level diagnostic program also.

   There needs to be a master and a slave side to all this so the master

   can tell the slave what's going wrong with the test.

5.2  SINGLE CONNECTION MEASUREMENTS

   Round trip delay times

      Time from moment the packet is sent by the TCP to the time that

      the ACK is received by the TCP.

      Time from the moment the USER issues the SEND to the time that the

      USER gets the successful return code.

         Note: packet size should be used to distinguish from one set of

         round trip times and another.

         Network destination, and current configuration and traffic load

         may also be issues of importance that must be taken into

         account.

         What if the destination TCP decides to queue up ACKs and send a

         single ACK after a while? How does this affect round trip

         statistics?

Cerf, Dalal & Sunshine                                         [Page 42]

RFC 675              Specification of Internet TCP         December 1974

         What about out of order arrivals and the bunched ACK for all of

         them?

         The histogram of round trip times include retransmission times

         and these must be taken into account in the analysis and

         evaluation of the collected data.

         Packet size statistics

      Histogram of packet length in both directions on the full duplex

      connection.

      Histogram of letter size in both directions.

   Measure of disorderly arrival

      Distance from the first octet of arriving packet to the left

      window edge. A histogram of this measure gives an idea of the out

      of order nature of packet arrivals. It will be 0 for packets

      arriving in order.

   Retransmission Histogram

   Effective throughput

      This is the effective rate at which the left edge of the window

      advances. The time interval over which the measure is made is a

      parameter of the measurement experiment. The shorter the interval,

      the more bursty we would expect the measure to be.

      It is possible to measure effective data throughput in both

      directions from one TCP by observing the rate at which the left

      window edge is moving on ACK sent and received for the two

      windows.

      Since throughput is largely dependent upon buffer allocation and

      window size, we must record these values also. Varying window for

      a fixed file transmission might be a good way to discover the

      sensitivity of throughput to window size.

   Output measurement

      The throughput measurement is for data only, but includes

      retransmission. The output rate should include all octets

      transmitted and will give a measure of retransmission overhead.

      Output rate also includes packet format overhead octets as well as

      data.

Cerf, Dalal & Sunshine                                         [Page 43]

RFC 675              Specification of Internet TCP         December 1974

   Utilization

      The effective throughput divided by the output rate gives a

      measure of utilization of the communication connection.

   Window and buffer allocation measurements

      Histogram of letters outstanding, measured at the instant of SEND

      receipt by TCP from user or at instant of arrival of a letter for

      a receiving user.

      Buffers in use on the SEND side upon packet departure into the

      net; buffers in use on the RECEIVE side upon delivery of packet

      into a USER Buffer.

5.3  MULTICONNECTION MEASUREMENTS

   Statistics on User Commands sent to the local TCP

   Statistics of error or success codes returned [histogram of each type

   of error or return response]

   Statistics of control bit use

      Counter for each control bit over all packets emitted by the TCP

      and another for packets accepted

   Count data carrying packets

   Count ACK packets with no data

   Error packets distribution by error type code received from the net

   and sent out into the net

5.4  MEASUREMENT IMPLEMENTATION PHILOSOPHY

   We view the measurement process as something which occurs internal to

   the TCP but which is controllable from outside. A well known socket

   owned by the TCP can be used to accept control which will select one

   or more measurement classes to be collected. The data would be

   periodically sent to a designated foreign socket which would absorb

   the data for later processing, in the manner currently used in the

   ARPANET IMPs. Each measurement class has its own data packet format

   to make the job of parsing and analyzing the data easier.

Cerf, Dalal & Sunshine                                         [Page 44]

RFC 675              Specification of Internet TCP         December 1974

   We would restrict access to TCP measurement control to a few

   designated sites [e.g. NMC, SU-DSL, BBN]. This is easily done by

   setting up listening control connections on partially specified

   foreign sockets.

6.  SCHEDULE OF IMPLEMENTATION

7.  REFERENCES

   1. CEKA74

      V. Cerf and R. Kahn, "A Protocol For Packet Network

      Intercommunication," IEEE Transactions on Communication, vol. C-

      2O, No. 5. May 1974, pp. 637-648.

   2. CERF74

      V. Cerf, "An Assessment of ARPANET Protocols," in Proceedings of

      the Jerusalem Conference on Information Technology, July l974

      [RFC#635, INWG Note # ***].

   3.CESU74

      V. Cerf and C. Sunshine, "Protocols and Gateways for the

      Interconnection of Packet Switching Networks," Proc. of the

      Subconference on Computer Nets, Seventh Hawaii International

      Conference on Systems Science, January 1974.

   4. HEKA70

      F. Heart, R.E. Kahn, et al, "The Interface Message Processor for

      the ARPA Computer Network," AFIPS 1970 SJCC Proceedings, vol. 36,

      Atlantic City, AFIPS Press, New Jersey, pp. 551-567.

   5. POUZ74

      L. Pouzin, "CIGALE, the packet switching machine of the CYCLADES

      computer network," Proceedings of the IFIP74 Congress, Stockholm,

      Sweden.

   6. ROWE74

      L. Roberts and B. Wessler, "Computer Network Development to

      achieve resource sharing," AFIPS 1970, SJCC Proceedings, vol. 36,

      Atlantic City, AFIPS Press, New Jersey, pp. 543-549.

Cerf, Dalal & Sunshine                                         [Page 45]

RFC 675              Specification of Internet TCP         December 1974

   7. POUZ73

      L. Pouzin, "Presentation and major design aspects of the CYCLADES

      Computer Network," Data Networks: Analysis and Design, Third Data

      Communications Symposium, St. Petersburg, Florida, November 1973,

      pp. 80-87.

   8. SCWI71

      R. Scantlebury and P.T. Wilkinson, "The Design of a Switching

      System to allow remote Access to Computer Services by other

      computers and Terminal Devices," Second Symposium on Problems in

      the Optimization of Data Communication Systems Proceedings, Palo

      Alto, California, 0ctober 1971, pp. 160-167.

   9. POST72

      J. Postel, "Official Initial Connection Protocol," Current Network

      Protocols, Network Information Center, Stanford Research

      Institute, Menlo Park, California. January 1972 (NIC 7101).

   10. CACR70

      C.S. Carr, S.D. Crocker, and V.G. Cerf, "Host-Host Communication

      Protocol in the ARPA Network," AFIPS Conference Proceedings, vol.

      36, 1970 SJCC, AFIPS Press, Montvale, N.J.

   11. ZIEL74

      H. Zimmerman and M. Elie, "Transport Protocol. Standard Host-Host

      Protocol for heterogeneous computer networks," INWG#61, April

      1974.

   12. CRHE72

      S. D. Crocker, J. F. Heafner, R. M. Metcalfe and J. B. Postel,

      "Function-oriented protocols for the ARPA Computer Network," AFIPS

      Conference Proceedings, vol. 41, 1972 FJCC, AFIPS Press, Montvale,

      N.J.

   13. DALA74

      Y. Dalal, "More on selecting sequence numbers," INWG Protocol Note

      #4, October 1974.

Cerf, Dalal & Sunshine                                         [Page 46]

RFC 675              Specification of Internet TCP         December 1974

   14. SUNS74

      C. Sunshine, "Issues in communication protocol design -- formal

      correctness." INWG Protocol Note #5, October 1974

   BELS74

      D. Belsnes, "Note on single message communication," INWG Protocol

      Note #3. September 1974.

   16. TOML74

      R. Tomlinson, "Selecting sequence numbers," INWG Protocol Note #2,

      September 1974.

   17. SCHA74

      R. Schantz, "Reconnection Protocol", private communication;

      available from Schantz at BBN.

   18. POUZ74A

      L. Pouzin, "A proposal for interconnecting packet switching

      networks, INWG Note #60, March 1974 [also submitted to EUROCOMP

      74].

   19. DLMG74

      D. Lloyd, M. Galland, and P. T. Kirstein, "Aims and objectives of

      internetwork experiments," to be published as an INWG Experiments

      Note.

   20. MCKE73

      A. McKenzie, "Host-Host Protocol for the ARPANET," NIC # 8246,

      Stanford Research Institute [also in ARPANET Protocols Notebook

      NIC 7104].

   21. BELS74A

      D. Belsnes, "Flow control in packet switching networks," INWG Note

      #63, October 1974.

Cerf, Dalal & Sunshine                                         [Page 47]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 1: TCB Major States

                              0-no TCB

      \____________________________________________________________/

                       OPEN    |    A   CLOSE           CLOSE    A

                    ---------- |    | ----------      ---------- |

                    set up TCB |    | remove TCB      remove TCB |

                               |    |                            |

                               |    |       collision retry,     |

        SYN arrives          __V____|__       SEND, INTER        |

       -------------        / S1=0     \    ----------------     |

       send SYN, ACK       |  S2=0 F=0  |       send SYN         |

     ______________________|  R=0  C=0  |_____________________   |

    |                      |  U=0/1     |                     |  |

    |                      |            |   SYN arrives       |  |

    |      error,timeout   |   1-OPEN   |   -----------       |  |

    |      -------------    \__________/    collision;        |  |

    |        clear TCB         A    A       set timeout       |  |

    |     _____________________|    |_____________________    |  |

  __V____|__                                             _|___V__|_

 / S1=1     \                                           / S1=1     \

|  S2=0 F=0  |                                         |  S2=0 F=0  |

|  R=1  C=0  |                       SYN, ACK arrives  |  R=0  C=0  |

|  U=0/1     |  ACK arrives          ----------------  |  U=0       |

|            |  -----------              send ACK      |            |

| 3-SYN rcvd |_________________       _________________| 2-SYN sent |

 \__________/                  |     |                  \__________/

    |                        __V_____V__

    |                       / S1=1      \

    |  CLOSE               |  S2=1 F=0   |

    | --------             |  R=1  C=0   |     FIN arrives

    | send FIN             |  U=0        | -------------------

    |                      |             | tell user, send FIN

    |      ________________|4-established|______________________

    |     |    CLOSE        \___________/                       |

    |     |   -------                                           |

  __V_____V_  send FIN                                   _______V__

 / S1=1     \                                           / S1=1     \

| S2=0/1 F=1 |     timeout or                          |  S2=1 F=1  |

|  R=1   C=1 | FIN, error, arrives            CLOSE    |  R=1  C=0  |

|  U=0/1     | -------------------          ---------- |  U=0       |

|            |     remove TCB               remove TCB |            |

| 5-FIN wait |_____________________       _____________| 6-FIN rcvd |

 \__________/                      |     |              \__________/

                                   |     |

       ____________________________V_____V_______________________

      /                                                          \

                                  0-no TCB

Cerf, Dalal & Sunshine                                         [Page 48]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 2.1: Structure of the TCP

      |       _____________            _______________       |

      |      |             |          |               |      |

      |      |             |          | INPUT PACKET  |<---->|

      |      | REASSEMBLER |          |    HANDLER    |      |

      |      |_____________|          |_______________|      |

      |             |_______________          |              |

      |                             |         |              |

      |       _________             |         |              |

      |      |         |          __V_________V____          |  NETWORK

      |<=====| SYSTEM  |         |                 |         |    or

      |      |  CALLS  |<========|       TCB's     |<========|   some

USERS |=====>|   or    |         |        and      |         |  NETWORK

      |      |  USER   |========>|ASSOCIATED QUEUES|========>| INTERFACE

      |<---->|INTERFACE|         |_________________|         |  PROGRAM

      |      |_________|            A         A              |

      |                             |         |              |

      |               ______________|         |              |

      |       _______|_____            _______|_______       |

      |      |             |          |               |      |

      |      | PACKETIZER  |          | OUTPUT PACKET |      |

      |      |             |          |    HANDLER    |<---->|

      |      |_____________|          |_______________|      |

      |                                                      |

     =======> Logical or physical flow of data (packets/letters)

     -------> "Interaction"

     NOTE:    The signalling of processes by others is not shown

Cerf, Dalal & Sunshine                                         [Page 49]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 2.2a:                               ________

Address Check                             / Begin  \

                                          \________/

                                               |

                                              _V_

                                           .'     '.

                                         .' packet  '.

                                       .'   foreign   '.

                  ___________________.'  socket matches '.

                 |                no  '.  a TCB local  .'

                 |                      '.   socket  .'

                 |                        '.   ?   .'

                 |                          '.___.'

                 |                             | yes

                 |                            _V_

                 |                         .'     '.

                 |                       .' packet  '.           ___

                 |                     .'local socket '.        /   \

                 |                   .'  matches fully  '.____\| YES |

                 |                    '. specified TCB .'     / \___/

                 |                      '.fgn socket .'

                 |                        '.   ?   .'

                _V_                         '.___.'

             .'     '.                         | no

           .'   SYN,  '.                      _V_

         .'FIN,INT,DSN, '.                 .'     '.

  _____.'or text length>0 './_____       .' matches '.

 |  no  '.   or QUERY    .' \     |    .'partly spec. '.

 |        '.           .'         |___.'  or unspec. TCB '.

 |          '.   ?   .'            no  '.     foreign   .'

 |            '.___.'                  '.   socket  .'

 |               | yes                   '.   ?   .'

 |     __________V_________                 '.___.'

 |    |                    |                   | yes

 |    |   Create error 7   |                  _V_

 |    | packet. Signal OPH |               .'     '.

 |    |____________________|             .' packet  '.

 |               |               ______.' has SYN set '.

 |           ____V____          |   no  '.           .'

 |          |         |         |         '.   ?   .'

 |_________\| discard |/________|          '.___.'

           /|_________|\                      |

                 |                           _V_

                _V_                         /   \

               /   \                       | YES |

              | NO  |                       \___/

               \___/

Cerf, Dalal & Sunshine                                         [Page 50]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 2.2b-1:                         _______

Input Packet Handler                  / Begin \

                                      \_______/

                                          |

 ________________________________________\|/_________________________

|                              A         /|\                         |

|                              |          |                          |

|                              |         _V_                         |

|                              |       .'   '.          _______      |

|                              |     .' input '.       | go to |     |

|                              |   .'  packet   '.____\| sleep |     |

|                              |    '.available.'  no /|_______|     |

|                              |      '.__?__.'                      |

|                              |          | yes                      |

|                              |         _V_                         |

|                              |       .'   '.                       |

|   .->SPECIAL FUNCT. Fig 4.7  |     .'address'.                     |

|   | .->ERR Fig 4.5,4.6       |___.' check OK  '.                   |

|   | | .->SYN Fig 4.1,4.2      no  '.    ?    .'                    |

|   | | | .->INT Fig 4.3              '._____.'                      |

|   | | | | .->FIN Fig 4.4                | yes              ________|_

|   | | | | |                            _V_                | discard  |

|  _|_|_|_|_|___________               .'   '.              |(or queue)|

|  |                    |            .' error '.            |__________|

|<-| Control Processing |/_________.'or control '.                   A

   |____________________|\     yes  '.    ?    .'                    |

        |                             '._____.'                      |

        | (INT with data)                 | no                       |

        |                                 |                          |

        V                                _V_                         |

      to "X"                           .'   '.              .        |

  in Fig 2.2b-2                      .'(estab)'.          .' '.      |

                              _____.' R=S1=S2=1 '.----->.'seq.#'.--->|

                             | yes  '.    ?    .'  no    '.OK .'  no |

                             |        '._____.'            '.'       |

                             |                              | yes    |

                             |    _______________           |        |

                             |   | Set S2=1, U=0 |          V        |

                             |   | Notify user   |         .'.       |

                             |<--| with event 2  |       .'ACK'.     |

                             |   | if U was 1    |<-----'.  OK .'--->'

                             |   |_______________|  yes   '. .'   no

                             |                              '

                             V

                           to "Y"

                       in Fig 2.2b-2

Cerf, Dalal & Sunshine                                         [Page 51]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 2.2b-2: Input Packet Handler (continued)

                            "Y"

                             |

           .'.              _V_

         .'txt'.          .'   '.        ______________________________

       .'lgth>0 '.      .'within '.     |Use ACK to advance send window|

,<----'. or DSN  .'<---'. window  .'--->|Release ACK'ed packets from   |

|  no   '.  ?  .'   no   '.  ?  .'  yes |retransmit or send queues. If |

|         '._.'            '._.'        |any packet had EB bit set     |

|           | yes                       |remove buffer from Packetized |

|   ________V____________________       |buffer queue and inform user  |

|  |Create ACK packet. Put on    |      |(success). Signal Packetizer. |

|<-|Send packet queue. Signal OPH|      |______________________________|

|  |_____________________________|                      |

|                                                       |

|          _____________________________________________|

|         |

|         |

|         |                      "X"

|         |                       |

|        _V_                     _V_              _____________________

|      .'   '.                 .'TCB'.           |Put packet on        |

|    .' text  '.   yes       .'Receive'.   yes   |Receive packet queue |

|  .' length>0  '.-------->.'  buffer   '.------>|in the right order.  |

|   '. or DSN  .'   A       '.available.'        |Signal Reassembler.  |

|     '.  ?  .'     |         '.  ?  .'          |_____________________|

|       '._.'       |           '._.'                           |

|         | no      |             | no                          |

|         |         |            _V_                            |

|________\|         |          .'   '.                          |

         /|         |        .' seq # '.         ________       |

          |         |      .' of packet '.  yes |Discard |      |

          |         |     '.  highest so .'---->|packet  |----->|

          |         |       '.   far   .'       |________|      |

          |         |         '.  ?  .'                         |

          |         |           '._.'                           |

          |         |             | no                          |

          |         |      _______V______________               |

          |         |     |Discard packet with   |              |

          |         |_____|highest seq. no from  |              |

          |               |Receive packet queue. |              |

          |               |______________________|              |

          |                                                     |

          |_____________________________________________________|

                                  |

                                  V

                      to "Begin" in Fig 2.2b-1

Cerf, Dalal & Sunshine                                         [Page 52]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 2.3-1: Reassembler

       _______

      / Begin \

      \_______/

          |

          |

          |<----------------------------------------------.

          |                      _____                    | yes

    ______V_____               .'     '.                 _|_

   |Get ready   |            .' Receive '.   yes       .'any'.

   |for next TCB|--------->.'Packet Queue '.-------->.' more  '.

   |____________|     A     '.  empty ?  .'     A     '.work?.'

                      |       '._______.'       |       '._.'

                      |            | no         |         | no

   "R"------>---------'          __V__          |     ____V____

                               .' is  '.        |    |  Go to  |

                             .' packet  '.      |    |  Sleep  |

  .--<----------------------'.DSN with no.'     |    |_________|

  |                     yes   '. data? .'       |

  |                             '.___.'         |

  |                                | no         |

  |                              __V__          |

  |                            .'     '.        |

  |                          .' Receive '.  yes |

  |                        .'Buffer Queue '.--->|

  |                         '.  empty ?  .'     |

  |  ________________         '._______.'       |

  | |Copy from packet|             | no         |<-------------"S"

  | |to buffer until |           __V__          |

  | |one is exhausted|         .'First'.        |

  | |Update receive  | yes   .' packet  '.   no |

  | |window.         |<----.'matches Recv '.--->'

  | |________________|      '.left window.'

  |         |                 '. edge ?.'

  |       __V__                 '.___.'

  |     .'Send '.

  |   .' Packet  '.   yes  _____________________________

  | .' Queue empty '.---->|Create ACK packet containing |

  |  '.     ?     .'      |new window. Signal OPH.      |

  |    '._______.'        |_____________________________|

  |      no |                            |

  |         |                            |

  |         '--------------------------->|

  |                                      |

  V                                      V

to "T"                                 to "U"

in Fig 2.3-2                        in Fig 2.3-2

Cerf, Dalal & Sunshine                                         [Page 53]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 2.3-2:  Reassembler (continued)

     "T"                                "U"

      |                                  |

      |                                  |           _____________

   ___V____           ___              __V__        |Mark progress|

  |process |  yes   .'   '.    yes   .'whole'.  no  |in packet.   |

  |  DSN   |<-----.'  DSN  '.<-----.' packet  '.--->|Return buffer|--->.

  |________|       '. set?.'        '.copied?.'     |to user.     |    |

      |              '._.'            '.___.'       |_____________|    |

      |                | no                                            |

      '--------------->|                                               |

                       |                                               |

                     __V__              __________________________     |

                   .' EOL '.  yes      |Return buffer to user.    |    |

                  '.  set? .'--------->|Return packet to free     |--->|

                    '.___.'            |storage. Signal Packetizer|    |

                    no |               |__________________________|    |

                       |                   A                           |

                     __V__                 |                           |

                   .' full'.               |                           |

                  '. buffer.'--------------'                           |

                    '.___.'   yes                                      |

                       | no                                            |

                       |                                               |

    ___________________V__________________                             |

   |Mark progress in buffer. Return packet|                            |

   |to free storage. Signal Packetizer.   |                   ,--------'

   |______________________________________|                   |

                       |                                      |

                       |                                      |

                       V                                      V

              to "R" in Fig 2.3-1                    to "S" in Fig 2.3-1

Cerf, Dalal & Sunshine                                         [Page 54]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 2.4: Packetizer

    _______               ________________________

   / Begin \____________\| Get ready for next TCB |/___________________

   \_______/            /|________________________|\                   |

                                      |                                |

                                    __V__               _____          |

                                  .'Send '.           .' any '.        |

                            no  .' Buffer  '.  yes  .'  more   '.  yes |

                 .-------------'.   Queue   .'---->'.   work    .'-----'

                 |               '.empty? .'   A     '.   ?   .'

     ____________V____________     '.___.'     |       '.___.'

    |Pick packet size depend- |                |          | no

,-->|ing on send buffer, TCB  |                |    ______V______

|   |buffer space, window, etc|                |   | go to sleep |

|   |_________________________|                |   |_____________|

|                |                             |

|              __V__                           |

|            .'Send '.                         |

|          .' window  '.  no                   |

|         '.has room ? .'--------------------->|

|           '._______.'                        |

|                | yes                         |

|              __V__                           |

|            .' TCB '.                         |

|          .' buffer  '.   no                  |

|        .'space avail- '.---------------------'

|         '.  able ?   .'                   A

|           '._______.'                     |

|                | yes                      |

|   _____________V____________     _________|_______     ____________

|  |Copy from Send buffer to  |   |Move buffer from |   |Set EOL bit |

|  |packet until packet full. |   |Send queue to    |<--|in packet   |

|  |Put packet on Send packet |   |packetized queue |   |header      |

|  |queue. Signal OPH.        |   |_________________|   |____________|

|  |__________________________|             A                  A

|                |                          | no               |

|              __V__                      __|__                |

|            .'whole'.                  .' EOL '.              |

|          .'  Send   '.  yes         .' set in  '.  yes       |

|         '.  buffer   .'----------->'.   Send    .'-----------'

|           '.copied?.'                '.buffer?.'

|             '.___.'                    '.___.'

|                | no

|   _____________V__________

|  |Note in TCB where in    |

 --|Send buffer we stopped. |

   |________________________|

Cerf, Dalal & Sunshine                                         [Page 55]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 2.5a:

Output Packet Handler

                                        _______

                                       / Begin \

                                       \_______/

                                           |

                                           |<--------------------------.

                               ____________V___________                |

                              | Get ready for next TCB |               |

                              |________________________|               |

                                      |                                |

,------------------------------------>|                                |

|                                   __V__               _____          |

|               _____             .'Send '.           .' any '.        |

|        yes  .' ACK '.     no  .' Buffer  '.  yes  .'  more   '.  yes |

|      .-----'.bit set.'<------'.   Queue   .'---->'.   work    .'-----'

|      |       '.___.'           '.empty? .'    A    '.   ?   .'

|      |       no |________        '.___.'      |      '.___.'

|      |                   |__________          |         | no

|  ____V__________________            |         |         |

| |Put latest receive left|   ________v______   |   ______V______

| |window edge in ACK.    |->|Transmit packet|  |  | go to sleep |

| |_______________________|  |_______________|  |  |_____________|

|                                     |         |

|     ________________              __V__       |

|    |Return packet to|           .'pckt '.     |_________________

|    |buffer pool as  |    no   .'seq # to '.                     |

|    |it has been     |<------.'rgt of Send  '.                   |

|    |ACKed           |        '.left window.'                    |

|    |________________|          '.  edge .'                      |

|             |                    '.___.'                        |

|             |                       | yes                       |

|             |        _______________V________________           |

|             |       |Move packet to retransmit queue;|          |

|             |       |set new retrans. time for it.   |          |

|             |       |________________________________|          |

|             |                       |                           |

|             '---------------------->|                           |

|                                   __V__                         |

|                            no   .'Time '.   yes                 |

 -------------------------------.'to switch'.---------------------'

                                 '.TCB's? .'

                                   '.___.'

Cerf, Dalal & Sunshine                                         [Page 56]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 2.5b:

Retransmit Process

                                _______

                               / Begin \

                               \_______/

                                   |

                                   |<----------------------------------.

                       ____________V___________                        |

                      | Get ready for next TCB |                       |

                      |________________________|                       |

                                   |                                   |

 .-------------------------------->|                                   |

 |                               __V__                                 |

 |                             .' Any '.                _____          |

 |                           .'packet's '.            .' any '.        |

 |                         .'retrans. time'.  no    .'  more   '.  yes |

 |                        '. has occurred  .'----->'.   work    .'-----'

 |                          '.  for this .'          '.   ?   .'

 |                            '. TCB ? .'              '.___.'

 |                              '.___.'                   |

 |                                 | yes                  | no

 |                                 |                ______V______

 |                         ________V________       | go to sleep |

 |                        |Move packet to   |      |_____________|

 '------------------------|Send Packet      |

                          |queue. Signal OPH|

                          |_________________|

Cerf, Dalal & Sunshine                                         [Page 57]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 3.1:

OPEN

                                 _______

                                / Begin \

                                \_______/

                                    |

                                  __V__

                                .'User '.          _______

                              .'permitted'.   no  |       |

                            .'  access to  '.---->|error 1|------------.

                             '.this local .'      |_______|            |

                               '.socket?.'                             |

                                 '.___.'                               |

                                    | yes                              |

                                  __V__                                |

                                .' fgn '.                              |

                         yes  .' socket  '.  no                        |

                       .-----'. specified .'----.                      |

                       |       '.   ?   .'      |                      |

                     __V__       '.___.'      __V__         _______    |

   _______         .'conn-'.                .'space'.  no  |       |   |

  |       |  yes .' ection  '.             '.for TCB.'---->|error 4|-->|

,-|error 6|<----'.  already  .'              '.___.'       |_______|   |

| |_______|       '.exists?.'                   | yes                  |

|                   '.___.'                     |                      |

|                      | no                 ____V__________            |

|   _______          __V__                 |Create TCB. Set|           |

|  |       |   no  .'space'.               |S1=S2=R=F=C=1  |           |

|<-|error 4|<-----'.for TCB.'              |Set U=1        |           |

|  |_______|        '.___.'                |_______________|           |

|                      | yes                       |                   |

|                      |                           |                   |

|             _________V__________                 |                   |

|            |Create TCB. Set U=0 |                |                   |

|            |Set S1=S2=R=F=C=1   |                |                   |

|            |____________________|                |                   |

|                      |                           |                   |

|                      '-------------.-------------'                   |

|                                    |                                 |

|               _____________________V__________________               |

|              |Return local connection name and Success|              |

|              |________________________________________|              |

|                                    |                                 |

 ----------------------------------->|<--------------------------------'

                                 ____V___

                                / Return \

                                \________/

Cerf, Dalal & Sunshine                                         [Page 58]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 3.2:

SEND

                   _______

                  / Begin \

                  \_______/

                      |

                    __V__

                  .'conn-'.

                .' ection  '.               _________

              .'  legal for  '.  no        |         |

             '. this process  .'---------->| error 1 |-----------.

               '.     ?     .'             |_________|           |

                 '._______.'                                     |

                      | yes                                      |

                    __V__                                        |

                  .'conn-'.                 _________            |

                .' ection  '.   no         |         |           |

              .'    open     '.----------->| error 3 |---------->|

               '.     ?     .'             |_________|           |

                 '._______.'                                     |

                      | yes                                      |

                    __V__                                        |

                  .' fgn '.                 _________            |

                .' socket  '.  no          |         |           |

               '. specified .'------------>| error 5 |---------->|

                 '.(U=0)? .'               |_________|           |

                   '.___.'                                       |

                      | yes                                      |

                    __V__                                        |

                  .'conn-'.                 _________            |

                .' ection  '.  yes         |         |           |

               '. closing ? .'------------>| error 12|---------->|

                 '.(F,C=1).'               |_________|           |

                   '.___.'                                       |

                      | no                                       |

  ____________________V________________________________          |

 |Put buffer on Send Buffer queue and signal Packetizer|         |

 |_____________________________________________________|         |

                      |                                          |

                      |<-----------------------------------------'

                  ____V___

                 / Return \

                 \________/

Cerf, Dalal & Sunshine                                         [Page 59]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 3.3:

INTERRUPT

                   _______

                  / Begin \

                  \_______/

                      |

                      |

                      V

                Same as SEND

                      |                                          |

                      |                                          |

  ____________________V_________________________                 |

 |Return any pending Send buffers with code 10. |                |

 |Create INT packet on outgoing packet queue.   |                |

 |Signal Output Packet Handler.                 |                |

 |______________________________________________|                |

                      |                                          |

                      |<-----------------------------------------'

                  ____V___

                 / Return \

                 \________/

Cerf, Dalal & Sunshine                                         [Page 60]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 3.4:

RECEIVE

                   _______

                  / Begin \

                  \_______/

                      |

                    __V__

                  .'conn-'.

                .' ection  '.               _________

              .'  legal for  '.  no        |         |

             '. this process  .'---------->| error 1 |-----------.

               '.     ?     .'             |_________|           |

                 '._______.'                                     |

                      | yes                                      |

                     _V_                                         |

                   .'   '.                                       |

                 .'       '.                                     |

               .'connection '.                                   |

             .'     state     '.                                 |

            :___________________:                   _________    |

               |      |      |                     |         |   |

           1-4 |  5,6 |    0 '-------------------->| error 3 |-->|

               |      '---------------------.      |_________|   |

     __________V__________                  |                    |

    |Put buffer on Receive|                 |       _________    |

    |Buffer queue. Signal |                 |      |         |   |

    |Reassembler          |                 '----->| error 12|-->|

    |_____________________|                        |_________|   |

               |                                                 |

               |<------------------------------------------------'

           ____V___

          / Return \

          \________/

Cerf, Dalal & Sunshine                                         [Page 61]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 3.5:

CLOSE

                   _______

                  / Begin \

                  \_______/

                      |

                    __V__

                  .'conn-'.

                .' ection  '.               _________

              .'  legal for  '.  no        |         |

             '. this process  .'---------->| error 1 |-----------.

               '.     ?     .'             |_________|           |

                 '._______.'                                     |

                      | yes                                      |

                     _V_                                         |

                   .'   '.                                       |

                 .'       '.                                     |

               .'connection '.                                   |

             .'     state     '.                                 |

            :___________________:                   _________    |

            5|   |3,4  |1,2,6  |0                  |         |   |

             |   |     |       '------------------>| error 3 |-->|

,------------'   |     '-------------------.       |_________|   |

|  ______________V______________________   |                     |

| |Return all buffers to user with error|  |     ___________     |

| |12; clear all packet queues, create  |  |    |Remove TCB |    |

| |FIN packet, signal Output Packet     |  '--->|Return     |--->|

| |Handler, set C=F=1                   |       |Success    |    |

| |_____________________________________|       |___________|    |

|                      |                                         |

 --------------------->|<----------------------------------------'

                   ____V___

                  / Return \

                  \________/

Cerf, Dalal & Sunshine                                         [Page 62]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 3.6:

STATUS

                   _______

                  / Begin \

                  \_______/

                      |

                    __V__

                  .'conn-'.

                .' ection  '.               _________

              .'  legal for  '.  no        |         |

             '. this process  .'---------->| error 1 |-----------.

               '.     ?     .'             |_________|           |

                 '._______.'                                     |

                      | yes                                      |

                    __V__                   __________           |

                  .'conn-'.                |Return    |          |

                .' ection  '.  no          |state=0 or|          |

               '.   open ?  .'------------>|error 3   |--------->|

                 '._______.'               |__________|          |

                      | yes                                      |

           ___________V___________                               |

          |Fill in reply from TCB.|                              |

          |Return Success to user.|                              |

          |_______________________|                              |

                      |                                          |

                      |<-----------------------------------------'

                  ____V___

                 / Return \

                 \________/

Cerf, Dalal & Sunshine                                         [Page 63]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 4.1:

SYN (no ACK)

                              _______

                             / Begin \

                             \_______/

                                 |

                                _V_

                              .'   '.

                            .'       '.

                          .' S1, S2, R '.

                        .'       ?       '.

                       :___________________: 1,1,1        _________

 __________             |     |     |     | (states 4-6) |         |

|Treat as a|      1,0,1 |     |     |     '------------->| error 6 |-->.

|duplicate.|<-----------'     |     |                    |_________|   |

|Retransmit|                  |     | 1.0,0                            |

|SYN, ACK  |            0,0,0 |     | (Syn sent)   ________________    |

|__________|      (listening) |     '------------>|Collision: Clear|   |

     |                        |                   |S1, set timeout,|   |

     |   _____________________V________________   |remove SYN from |-->|

     |  |Set R=S1=1. If U=1 set foreign socket |  |retransmit queue|   |

     |  |in TCB to match packet local socket.  |  |________________|   |

     |  |Send SYN, ACK. Signal OPH. Fill in TCB|                       |

     |  |with send window, receive sequence #. |                       |

     |  |______________________________________|                       |

     |                        |                                        |

     |                        |                                        |

     '----------------------->|<---------------------------------------'

                           ___V__

                          / Done \

                          \______/

Cerf, Dalal & Sunshine                                         [Page 64]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 4.2:

SYN,ACK

                        _______

                       / Begin \

                       \_______/

                           |

                         __V__

                       .'     '.

                     .' State 2 '.  no

                    '.S1=1;S2=R=0.'----------------.

                      '.   ?   .'                  |

                        '.___.'                    |

                           | yes                   |

                         __V__              _______V______

                       .' ACK '.   no      |              |

                     .' correct '.-------->| send error 6 |

                      '.   ?   .'          |______________|

                        '.___.'                    |

                           | yes                   |

                  _________V_________              |

                 |Set S2=R=1. Process|             |

                 |ACK. Send ACK.     |             |

                 |___________________|             |

                           |                       |

                           |<----------------------'

                        ___V__

                       / Done \

                       \______/

Cerf, Dalal & Sunshine                                         [Page 65]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 4.3:

INT (from net)

                   _______       ____________

                  / Begin \____\|Process ACK |

                  \_______/    /|(may set S2)|------.

                                |____________|      |

                                                    |

                                                  __V__

                        ____________            .' in  '.

                       | Discard    |     no  .' state 4 '.

              .<-------| (or queue) |<-------'. S1=S2=R=1 .'

              |        |____________|          '. F=0 ? .'

              |                                  '.___.'

              |                                     | yes

              |                                   __V__

              |         ____________            .'     '.

              |        | ACK and    |     no  .' within  '.

              |<-------| discard    |<-------'.  window   .'

              |        |____________|          '.   ?   .'

              |                                  '.___.'

              |                                     | yes

              |         ____________________________V_______________

              |        |Move Receive Left window edge to sequence   |

              |        |number of INT. Return event 10 with any     |

              |        |pending Receive buffers. Ruturn event 11 to |

              |        |user. Send ACK for INT.                     |

              |        |____________________________________________|

              |                                     |

              |                                   __V__

              |                 see       yes   .'data '.

              |              Figure<----------.' in this '.

              |                 2.2            '.packet?.'

              |                                  '.___.'

              |                                     | no

              '------------------------------------>|

                                                 ___V__

                                                / Done \

                                                \______/

Cerf, Dalal & Sunshine                                         [Page 66]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 4.4:

FIN

                 _______       ____________

                / Begin \____\|Process ACK |

                \_______/    /|(may set S2)|------.

                              |____________|      |

                                                  |

                                                __V__

                                              .'     '.

                                        no  .'S1=S2=R=1'.

                            .--------------'.  (estab-  .'

                            |                '.lished).'

                            |                  '.___.'

                            |                     | yes

                            |                   __V__

                      ______V_____            .'     '.

                     |            |     no  .' within  '.

   .-----------------| discard    |<-------'.  window   .'

   |                 |____________|          '.   ?   .'

   |                                           '.___.'

   |                                              | yes

   |                                            __V__

   |                             (state 4) 0  .'F bit'.  1 (state 5)

   |                            .------------'. value .'------------.

   |                            |              '.___.'              |

   |   _________________________V________                           |

   |  |Return all user buffers (event 12)|     _____________________V__

   |  |Clear all packet queues. Send FIN |    |Return success to User's|

   |  |packet. Set F=1. Inform user      |    |CLOSE.  Remove TCB.     |

   |  |"connection closing" (event 12)   |    |________________________|

   |  |__________________________________|                 |

   |                  |                                    |

   '----------------->|<-----------------------------------'

                   ___V__

                  / Done \

                  \______/

Cerf, Dalal & Sunshine                                         [Page 67]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 4.5:

Error 6 (bad SYN)

                _______

               / Begin \

               \_______/

                   |

                   |

                 __V__

               .'     '.

             .'refers to'.

           .'current pckt?'.                      _________

         .'(ACK matches seq '.  no               |         |

        '.  # of packet on   .'----------------->| discard |-----------.

          '.retrans or send.'                    |_________|           |

            '.  queues?) .'                                            |

              '._______.'                                              |

                   | yes                                               |

                   |                                                   |

                  _V_                                                  |

                .'   '.   1 (state 3)                                  |

              .' value '.--------------------------------.             |

               '. of R.'  bad SYN,ACK                    |             |

                 '._.'                                   |             |

                   |                                     |             |

                   | 0 (state 2)                         |             |

                   | bad SYN                             |             |

 __________________V__________________            _______V______       |

|Other side is established. Send RESET|          |Clear S1, R   |      |

|(put error packet's seq. # in ACK    |          |Remove SYN,ACK|      |

|field. Return all user buffers with  |          |from retrans  |      |

|code 14. Inform user with event 14   |          |queue.        |      |

|_____________________________________|          |______________|      |

                   |                                     |             |

                   |                                     V             |

                   |<--------------------------------------------------'

                ___V__

               / Done \

               \______/

Cerf, Dalal & Sunshine                                         [Page 68]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 4.6:

Error 7,8

                   _______

                  / Begin \

                  \_______/

                      |

                    __V__

                  .'     '.

                .'refers to'.                     _________

              .'   current   '.  no              |         |

             '. packet (check .'---------------->| discard |-----------.

               '.   ACK)?   .'         A         |_________|           |

                 '._______.'           |                               |

                      | yes            |                               |

                     _V_               |                               |

                   .'   '.             |                               |

                 .'       '.           |                               |

               .'connection '.         |                               |

             .'     state     '.       |                               |

            :___________________:      |                               |

           4|   5|   3|   2|   6|      |                               |

    .-------'    |    |    |    '------'                               |

    |            |    |    '-----------------------------.             |

    |            |    '-------------.                    |             |

    |            |                  |                    |             |

 ___V___     ____V_______     ______V_______     ________V_________    |

|Pass to|   |Remove TCB. |   |Clear S1, R.  |   |Discard. SYN will |   |

|user   |   |Return      |   |Remove SYN,ACK|   |be retrans to     |   |

|_______|   |success to  |   |from transmit |   |avoid receiver    |   |

    |       |user's CLOSE|   |queue (go to  |   |having to queue it|   |

    |       |____________|   |state 1).     |   |__________________|   |

    |            |           |______________|            |             |

    |            V                  |                    V             |

    '------------------------------>|<---------------------------------'

                                 ___V__

                                / Done \

                                \______/

Cerf, Dalal & Sunshine                                         [Page 69]

RFC 675              Specification of Internet TCP         December 1974

FIGURE 4.7:

RESET

                                _______

                               / Begin \

                               \_______/

                                   |

                                 __V__

                           no  .'Reset'.  yes

                 .------------'. All ? .'------------------.

                 |              '.___.'                    |

                 |                                _________V_________

                 |                               |Clear all TCB's for|

                 |                               |foreign TCP. Inform|

                 |                               |users with event 14|

                 |                               |___________________|

               __V__                                       |

             .' Is  '.             _________               |

           .'  RESET  '.   no     |         |              |

         .'believable ? '.------->| discard |------------->|

          '.(check ACK .'         |_________|              |

            '.field) .'                                    |

              '.___.'                                      |

                 | yes                                     |

 ________________V________________                         |

|Clear all queues for this TCB.   |                        |

|Return event 14 for user buffers.|                        |

|Inform User with event 14.       |                        |

|_________________________________|                        |

                 |                                         |

                 |<----------------------------------------'

              ___V__

             / Done \

             \______/

       [ This RFC was put into machine readable form for entry ]

       [ into the online RFC archives by Alex McKenzie with    ]

       [ support from GTE, formerly BBN Corp.           2/2000 ]

Cerf, Dalal & Sunshine                                         [Page 70]

